- CODE TABLES AND FLAG TABLES ASSOCIATED WITH BUFR/CREX TABLE B (Version 12-14/07/2005)

Notes:	In developing code tables associated with BUFR/CREX Table B to specify units of elements, the following principles should be applied:

(a)	Code tables specifying the units for an element which is defined, in the Manual on Codes, by a single symbolic letter shall be compatible with the relevant existing WMO code tables;

(b)	Code tables combining two or more existing WMO code tables to specify the units for an element which is defined, in the Manual on Codes, by a group of symbolic letters shall be compatible with the combined code figures of the relevant group of symbolic letters;

(c)	Code tables combining two or more existing WMO code tables to specify the units for an element which is defined, in the Manual on Codes, by different symbolic letters shall be compatible with the code figures of the relevant symbolic letters, with successive tens or hundreds values added, as appropriate;

(d)	Code tables and flag tables should only be used for reporting qualitative information. Quantitative information should be reported as observed using entries in Table 13."Data description operators" from Table C should be applied when a "scale change" or "data width change" is required;

(e)	Reference to existing specification(s) and code table(s) in the Manual on Codes, with explanation of possible deviations, shall be given in an additional table annexed to the code tables associated with BUFR/CREX Table B.

0 01 003

- WMO Region number/geographical area

| Code figure |  |
| --- | --- |
| 0 | Antarctica |
| 1 | Region I |
| 2 | Region II |
| 3 | Region III |
| 4 | Region IV |
| 5 | Region V |
| 6 | Region VI |
| 7 | Missing value |

0 01 007

- Satellite identifier

(See common Code table C-5 Part C/c.)

0 01 031

- Identification of originating/generating centre

(See common Code table C-1 in Part C/c.)

0 01 033

- Identification of originating/generating centre

(See common Code table C-1 in Part C/c.)

0 01 034

- Identification of originating/generating sub-centre

(To be defined by centres themselves)

(See common Code table C-12 in Part C/c.)

0 01 036

Agency in charge of operating the observing platform

(the first 3 digits represent the ISO country code)

| Code figure |  |
| --- | --- |
| 0-036000 | Reserved |
| 036001 | Australia, Bureau of Meteorology (BOM) |
| 036002 | Australia, Joint Australian Facility for Ocean Observing Systems (JAFOOS) |
| 036003 | Australia, the Commonwealth Scientific and Industrial Research Organisation (CSIRO) |
| 036004-124000 | Reserved |
| 124001 | Canada, Marine Environmental Data Service (MEDS) |
| 124002 | Canada, Institute of Ocean Sciences (IOS) |
| 124003-156000 | Reserved |
| 156001 | China, The State Oceanic Administration |
| 156002 | China, Second Institute of Oceanography State Oceanic Administration |
| 156003 | China, Institute of Ocean Technology |
| 156004-250000 | Reserved |
| 250001 | France, Institut de Recherche pour le Développement (IRD) |
| 250002 | France, Institut Français de Recherche pour l'Exploitation de la mer (IFREMER) |
| 250003-276000 | Reserved |
| 276001 | Germany, Bundesamt fuer Seeschiffahrt und Hydrographie (BSH) |
| 276002 | Germany, Institut fuer Meereskunde, Kiel |
| 276003-356000 | Reserved |
| 356001 | India, National Institute of Oceanography (NIO) |
| 356002 | India, National Institute for Ocean Technology (NIOT) |
| 356003 | India, National Centre for Ocean Information Service |
| 356004-392000 | Reserved |
| 392001 | Japan, Japan Meteorological Agency (JMA) |
| 392002 | Japan, Frontier Observational Research System for Global Change |
| 392003 | Japan, Japan Marine Science and Technology Centre (JAMSTEC) |
| 392004-410000 | Reserved |
| 410001 | Korea Rep., Seoul National University |
| 410002 | Korea Rep., Korea Ocean Research and Development Institute (KORDI) |
| 410003 | Korea Rep., Meteorological Research Institute |
| 410004-540000 | Reserved |
| 540001 | New Caledonia, Institut de Recherche pour le Développement (IRD) |
| 540002-554000 | Reserved |
| 554001 | New Zealand, National Institute of Water and Atmospheric Research (NIWA) |
| 554002-64300 | Reserved |
| 643001 | Russia, State Oceanographic Institute of Roshydromet |
| 643002 | Russia, Federal Service for Hydrometeorology and Environmental Monitoring |
| 643003-724000 | Reserved |
| 724001 | Spain, Instituto Español de Oceanografia |
| 724002-826000 | Reserved |
| 826001 | United Kingdom, Hydrographic Office |
| 826002 | United Kingdom, Southampton Oceanography Centre (SOC) |
| 826003-840000 | Reserved |
| 840001 | USA, NOAA Atlantic Oceanographic and Meteorological Laboratories (AOML) |

| 840002 | USA, NOAA Pacific Marine Environmental Laboratories (PMEL) |
| --- | --- |
| 840003 | USA, Scripps Institution of Oceanography (SIO) |
| 840004 | USA, Woods Hole Oceanographic Institution (WHOI) |
| 840005 | USA, University of Washington |
| 840006 | USA, Naval Oceanographic Office |
| 840007-1048574 | Reserved |
| 1048575 | Missing value |

| 0 01 090 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Technique for making up initial perturbation s |  |  |  |  |
| Code figure |  |  |  |  |
| 0 |  | LAF (Lagged-Average Forecasting) |  |  |
| 1 |  | Breeding |  |  |
| 2 |  | Singular vectors |  |  |
| 3 |  | Multiple analysis cycles |  |  |
| 4-191 |  | Reserved |  |  |
| 192-254 |  | Reserved for local use |  |  |
| 255 |  | Missing value |  |  |
| 0 01 092 |  |  |  |  |
| Type of ensemble forecast t |  |  |  |  |
| Code figure |  |  |  |  |
| 0 |  | Unperturbed high-resolution control forecast |  |  |
| 1 |  | Unperturbed low-resolution control forecast |  |  |
| 2 |  | Negatively perturbed forecast |  |  |
| 3 |  | Positively perturbed forecast |  |  |
| 4-191 |  | Reserved |  |  |
| 192-254 |  | Reserved for local use |  |  |
| 255 |  | Missing value |  |  |

0 02 001

- Type of station

| Code figure |  |
| --- | --- |
| 0 | Automatic station |
| 1 | Manned station |
| 2 | Hybrid:Both Manned and Automatic |
| 3 | Missing value |

0 02 002

- Type of instrumentation for wind measurement

| Bit No | Type of Instrumentation and original units for wind measurement |
| --- | --- |
| 0 | (measured in m s-1 unless otherwise indicated) |
| 1 | Certified Instruments |
| 2 | Originally measured in knots |
| 3 | Originally measured in kmh-1 |
| All 4 | Missing value |

0 02 003

- Type of measuring equipment used

| Code figure |  |
| --- | --- |
| 0 | Pressure Instrument associated with wind measuring equipment |
| 1 | Optical theodolite |
| 2 | Radio theodolite |
| 3 | Radar |
| 4 | VLF-Omega |
| 5 | Loran-C |
| 6 | Wind profiler |
| 7 | Satellite navigation |
| 8 | Radio-Acoustic Sounding System (RASS) |
| 9 | Sodar |
| 10-13 | Reserved |
| 14 | Pressure instrument associated with wind measuring equipment but pressure element failed during ascent |
| 15 | Missing value |

0 02 004

Type of instrumentation for evaporation measurement or type of crop for which evapotranspiration is reported

| Code figure | Instrumentation or crop type | Type of data |
| --- | --- | --- |
| 0 | USA open pan evaporimeter (without cover) | Evaporation |
| 1 | USA open pan evaporimeter (mesh covered) |  |
| 2 | GGI-3000 evaporimeter (sunken) |  |
| 3 | 20 m2 tank |  |
| 4 | Others |  |
| 5 | Rice | Evapotranspiration |
| 6 | Wheat |  |
| 7 | Maize |  |
| 8 | Sorghum |  |
| 9 | Other crops |  |
| 10-14 | Reserved |  |
| 15 | Missing value |  |

0 02 011

- Radiosonde type

(See common code table C-2 in part C/c.)

0 02 012

- Radiosonde computational method

(To be developed)

0 02 013

- Solar and Infrared radiation correction

| Code figure |  |
| --- | --- |
| 0 | No correction |
| 1 | CIMO solar corrected and CIMO infrared corrected |
| 2 | CIMO solar corrected and infrared corrected |
| 3 | CIMO solar corrected only |
| 4 | Solar and infrared corrected automatically by radiosonde system |
| 5 | Solar corrected automatically by radiosonde system |
| 6 | Solar and Infrared corrected as specified by country |
| 7 | Solar corrected as specified by country |
| 8-14 | Reserved |
| 15 | Missing value |

0 02 014

- Tracking technique/status of system used

(See common Code table C-7 in Part C/c.)

0 02 015

- Radiosonde completeness

| Code figure |  |
| --- | --- |
| 0 | Reserved |
| 1 | Pressure only radiosonde |
| 2 | Pressure only radiosonde plus transponder |
| 3 | Pressure only radiosonde plus radar reflector |
| 4 | No-pressure radiosonde plus transponder |
| 5 | No-pressure radiosonde plus radar reflector |
| 6-14 | Reserved |
| 15 | Missing value |

0 02 016

# Radiosonde configuration

| Bit No. |  |
| --- | --- |
| 1 | Train regulator |
| 2 | Light unit |
| 3 | Parachute |
| 4 | Rooftop release |
| All 5 | Missing value |

0 02 019

Satellite instruments

(See common Code table C-8 in Part C/c.)

0 02 020

- Satellite classification

| Code figure |  |
| --- | --- |
| 0 | Nimbus |
| 1 | VTPR |
| 2 | Tiros 1 (Tiros, NOAA-6 to NOAA-13) |
| 3 | Tiros 2 (NOAA-14 onwards) |
| 10 | EOS |
| 31 | DMSP |
| 61 | EUMETSAT Polar System (EPS) |
| 91 | ERS |
| 121 | ADEOS |
| 241 | GOES |
| 261 | JASON |
| 271 | GMS |
| 272 | MTSAT |
| 301 | INSAT |
| 331 | METEOSAT Operational Programme (MOP) |
| 332 | METEOSAT Transitional Programme (MTP) |
| 333 | METEOSAT Second Generation Programme (MSG) |
| 351 | GOMS |
| 380 | FY-1 |
| 381 | FY-2 |
| 382-400 | Reserved |
| 401 | GPS |
| 402 | GLONASS |
| 403 | GALILEO |
| 404-510 | Reserved |
| 511 | Missing value |

0 02 021

- Satellite Instrument data used In processing

| Bit No |  |
| --- | --- |
| 1 | High-resolution Infrared sounder (HIRS) |
| 2 | Microwave sounding unit (MSU) |
| 3 | Stratospheric sounding unit (SSU) |
| 4 | AMI (Advanced microwave instrument) Wind mode |
| 5 | AMI (Advanced microwave instrument) Wave mode |
| 6 | AMI (Advanced microwave instrument) Image mode |
| 7 | RADAR altimeter |
| 8 | ATSR (Along Track Scanning Radiometer) |
| All 9 | Missing value |

0 02 022

- Satellite data-processing technique used

- Bit flags denoting the elements Included in processing sounding data.

| Bit No. |  |
| --- | --- |
| 1 | Processing technique not defined |
| 2 | Automated statistical regression |
| 3 | Clear path |
| 4 | Partly cloudy path |
| 5 | Cloudy path |
| 6-7 | Reserved |
| All 8 | Missing value |

Notes:

(1)	Clear path means the sounding has been generated from clear radiances derived from actual clear spot measurements. Tropospheric and stratospheric HIRS data, as well as MSU and SSU data, have been used.

(2) 	Partly cloudy path means the sounding has been generated from clear radiances which have been calculated from partly cloudy spots. Tropospheric and stratospheric HIRS data, as well as MSU and SSU data, have been used.

(3) 	Cloudy path means the sounding has been generated only from stratospheric HIRS data, MSU data and SSU data. Tropospheric HIRS data have not been used because of cloudy conditions.

0 02 023

- Satellite Derived Wind Computation Method

| Code Figure |  |
| --- | --- |
| 0 | Reserved |
| 1 | Wind derived from cloud motion observed in the infrared channel |
| 2 | Wind derived from cloud motion observed in the visible channel |
| 3 | Wind derived from motion observed in the water vapour channel |
| 4 | Wind derived from motion observed in a combination of spectral channels |
| 5 | Wind derived from motion observed in the water vapour channel in clear air |
| 6 | Wind derived from motion observed in the ozone channel |
| 7 | Wind derived from motion observed in water vapour channel (cloud or clear air not specified) |
| 8-12 | Reserved |
| 13 | Root mean square |
| 14 | Reserved |
| 15 | Missing value |

0 02 024

- Integrated mean humidity computational method

| Code figure |  |
| --- | --- |
| 0 | Reserved |
| 1 | Table with full range of humidity variation in layer |
| 2 | Regression technique on 2 humidity values in layer |
| 3-14 | Reserved |
| 15 | Missing value |

0 02 025

- Satellite channel(s) used in computation

Bit flags denoting the Instrument and/or channels used in obtaining various physical parameters. If, In any grouping of parameters, all bits = 0, then no retrieval was made for that parameter or set of parameters.

| Bit No | Instrument (channels) |
| --- | --- |
| 1 | Reserved Group 1 - Layer precipitable water for the layers: surface to 700 hPa, 700 to 500 hPa, and 500 to 300 hPa |
| 2 | HIRS |
| 3 | MSU |
| 4-5 | Reserved Group 2 - Tropopause temperature and pressure |
| 6 | HIRS |
| 7 | MSU |
| 8-9 | Reserved Group 3 - Total ozone |
| 10 | HIRS (1,2,3,8,9,16,17) |
| 11 | HIRS (1, 2, 3, 9, 17) |
| 12 | MSU |
|  | Reserved Group 4 - Mean temperature for the layers: surface to 850 hPa, 850 to 700 hPa, 700 to 500 hPa, 500 to 400 hPa, 400 to 300 hPa, 300 to 200 hPa, and 200 to 100 hPa |
| 15 | HIRS |
| 16 | HIRS* |
| 17 | MSU |
| 18 | SKINTK (ocean only) |
|  | Reserved Group 5 - Channel combinations used to obtain mean temperatures for the layers 100 to 70 hPa, 70 to 50 hPa, 50 to 30 hPa, 30 to 10 hPa, 10 to 5 hPa, 5 to 2 hPa, 2 to 1 hPa, 1 to 0.4 hPa |
| 21 | HIRS* |
| 22 | SSU |
| 23 | MSU (3,4) |
|  | Reserved |
| All 25 | Missing value |

Note: HIRS* is equivalent to: HIRS channel:	1	(669 cm-1)

2	(679cm-1)

3	(690cm-1)

4	(2358cm-1)

0 02 030

- Method of current measurement

| Code Figure |  |
| --- | --- |
| 0 | Reserved |
| 1 | ADCP (Acoustic Doppler Current Profiler)* |
| 2 | GEK (Geomagnetic ElectroKinetograph) |
| 3 | Ship's set and drift determined by fixes 3-6 hours apart |
| 4 | Ship's set and drift determined by fixes more than 6 hours but less than 12 hours apart |
| 5 | Drift of buoy |
| 6 | ADCP (Acoustic Doppler Current Profiler) |
| 7 | Missing value |

Note: * Value deprecated.  Code figure 6 should be used instead.

0 02 031

- Duration and Time of current measurement

| Code Figure |  |  |
| --- | --- | --- |
| 0 | Reserved |  |
| 1 | Instantaneous |  |
| 2 | Averaged over 3 minutes or less | between H-1 and H |
| 3 | Averaged over more than 3 minutes, but 6 at the most |  |
| 4 | Averaged over more than 6 minutes, but 12 at the most |  |
| 5 | Instantaneous | between H-2 and H-1 |
| 6 | Averaged over 3 minutes or less |  |
| 7 | Averaged over more than 3 minutes, but 6 at the most |  |
| 8 | Averaged over more than 6 minutes, but 12 at the most |  |
| 9 | Vector or Doppler current profiling method not used |  |
| 10 | Reserved |  |
| 11 | 1 hour or less |  |
| 12 | More than 1 hour but 2 at the most |  |
| 13 | More than 2 hours but 4 at the most |  |
| 14 | More than 4 hours but 8 at the most |  |
| 15 | More than 8 hours but 12 at the most |  |
| 16 | More than 12 hours but 18 at the most |  |
| 17 | More than 18 hours but 24 at the most |  |
| 18 | Reserved |  |
| 19 | Drift method not used |  |
| 20-30 | Reserved |  |
| 31 | Missing value |  |

Notes:

(1)	Code figures 1-9: Duration and time of current measurement (vector or Doppler current profiling method).

(2)	Code figures 11-19: Period of current measurement (drift method).

(3)	H = Time of observation.

0 02 032

- Indicator for digitization

| Code figure |  |
| --- | --- |
| 0 | Values at selected depths (data points fixed by the instrument or selected by any other method) |
| 1 | Values at selected depths (data points taken from traces at significant depths) |
| 2 | Reserved |
| 3 | Missing value |

0 02 033

- Method of salinity/depth measurement

| Code Figure |  |
| --- | --- |
| 0 | No salinity measured |
| 1 | In situ sensor, accuracy better than 0.02 %o |
| 2 | In situ sensor, accuracy less than 0.02 %o |
| 3 | Sample analysis |
| 4-6 | Reserved |
| 7 | Missing value |

0 02 034

- Drogue type

| Code Figure |  |
| --- | --- |
| 0 | Unspecified drogue |
| 1 | Holey sock |
| 2 | TRISTAR |
| 3 | Window shade |
| 4 | Parachute |
| 5 | Non-Lagrangian sea anchor |
| 6-30 | Reserved (to be developed) |
| 31 | Missing value |

0 02 036

- Buoy type

| Code Fgure |  |
| --- | --- |
| 0 | Drifting buoy |
| 1 | Fixed buoy |
| 2 | Sub-surface float (moving) |
| 3 | Missing value |

0 02 037

- Method of tidal observation

| Code Figure |  |
| --- | --- |
| 0 | Reserved |
| 1 | Manual reading from vertical tide staff |
| 2 | Manual reading from single automatic recorder at station |
| 3 | Manual reading from multiple automatic recorders at station |
| 4 | Automatic reading from single automatic recorder at station without level reference check. |
| 5 | Automatic reading from a single automatic recorder at station with level reference check, or from multiple automatic recorders. |
| 6 | Reserved |
| 7 | Missing value |

0 02 038

- Method of water temperature and/or salinity measurement

| Code Figure |  |
| --- | --- |
| 0 | Ship intake |
| 1 | Bucket |
| 2 | Hull contact sensor |
| 3 | Reversing Thermometer |
| 4 | STD/CTD sensor |
| 5 | Mechanical BT |
| 6 | Expendable BT |
| 7 | Digital BT |
| 8 | Thermistor chain |
| 9 | Infra-red scanner |
| 10 | Micro-wave scanner |
| 11-14 | Reserved |
| 15 | Missing value |

0 02 039

- Method of wet bulb temperature measurement

| Code Figure |  |
| --- | --- |
| 0 | Measured wet-bulb temperature |
| 1 | Iced bulb measured wet-bulb temperature |
| 2 | Computed wet-bulb temperature |
| 3 | Iced bulb computed wet-bulb temperature |
| 4-6 | Reserved |
| 7 | Missing value |

0 02 040

- Method of removing code velocity and motion of platform from current

| Code Figure |  |  |
| --- | --- | --- |
| 0 | Ships motion removed by avaraging | Ship's velocity removed by botton tracking |
| 1 | Ships motion removed by motion compensation |  |
| 2 | Ships motion not removed |  |
| 3 | Ships motion removed by averaging | Ship's velocity removed by navigation |
| 4 | Ships motion removed by motion compensation |  |
| 5 | Ships motion not removed |  |
| 6 | Doppler current profiling method not used |  |
| 7-14 | Reserved |  |
| 15 | Missing value |  |

0 02 041

- Method for estimating reports related to synoptic features

| Code figure |  |
| --- | --- |
| 0 | Information based on manual analysis |
| 1 | Information based on computer analysis |
| 2 | Information based on data assimilation |
| 3 | Information based on computer analysis or data assimilation manually modified |
| 4-9 | Reserved |
| 10 | Information based on the numerical weather prediction |
| 11-62 | Reserved for future use |
| 63 | Missing value |

0 02 044

- Indicator for method of calculation of spectral wave data

| Code Figure |  |
| --- | --- |
| 0 | Reserved for future use |
| 1 | Longuet-Higgins (1964) |
| 2 | Longuet-Higgins (F3 method) |
| 3 | Maximum likelihood method |
| 4 | Maximum entropy method |
| 5-14 | Reserved |
| 15 | Missing value |

0 02 045

- Indicator for type of platform

| Code Figure |  |
| --- | --- |
| 0 | Sea station |
| 1 | Automatic data buoy |
| 2 | Aircraft |
| 3 | Satellite |
| 4-14 | Reserved |
| 15 | Missing value |

0 02 046

- Wave measurement instrumentation

| Code Figure |  |
| --- | --- |
| 0 | Reserved for future use |
| 1 | Heave sensor |
| 2 | Slope sensor |
| 3-14 | Reserved |
| 15 | Missing value |

0 02 048

- Satellite sensor indicator

| Code Figure |  |
| --- | --- |
| 0 | HIRS |
| 1 | MSU |
| 2 | SSU |
| 3 | AMSU-A |
| 4 | AMSU-B |
| 5 | AVHRR |
| 6 | SSMI |
| 7 | NSCAT |
| 8 | SEAWINDS |
| 9 | POSEIDON altimeter |
| 10 | JMR (Jason Microwave Radiometer) |
| 11-14 | Reserved |
| 15 | Missing value |

0 02 049

- Geostationary satellite data-processing technique used

| Bit No. |  |
| --- | --- |
| 1 | Processing technique not defined |
| 2 | Simultaneous physical retrieval |
| 3 | Clear sounding |
| 4 | Cloudy sounding |
| 5-7 | Reserved for future use |
| All 8 | Missing value |

Notes:

- Clear sounding indicates the sounding has been generated from a set of clear radiances using all available sounder radiances.

- Cloudy sounding indicates that sufficient clear radiances could not be identified in the sounding area. The sounding is calculated from the cloud top (cloud pressure greater than or equal to 70 hPa) upwards.

0 02 050

- Geostationary sounder satellite channels used

| Bit No. | Channel | Central wavelength (micrometers |
| --- | --- | --- |
| 1 | 1 | 14.71 |
| 2 | 2 | 14.37 |
| 3 | 3 | 14.06 |
| 4 | 4 | 13.64 |
| 5 | 5 | 13.37 |
| 6 | 6 | 12.66 |
| 7 | 7 | 12.02 |
| 8 | 8 | 11.03 |
| 9 | 9 | 9.71 |
| 10 | 10 | 7.43 |
| 11 | 11 | 7.02 |
| 12 | 12 | 6.51 |
| 13 | 13 | 4.57 |
| 14 | 14 | 4.52 |
| 15 | 15 | 4.45 |
| 16 | 16 | 4.13 |
| 17 | 17 | 3.98 |
| 18 | 18 | 3.74 |
| 19 | 19 | 0.969 |
| All 20 | Missing value |  |

Note:	Beginning with the first bit position (high order bit), if the bit position is set to one, then the channel is used. If the bit position is set to zero, then the channel is not used.

0 02 051

- Indicator to specify observing method for extreme temperatures

| Code figure |  |
| --- | --- |
| 0 | Reserved |
| 1 | Maximum/minimum temperatures |
| 2 | Automated instruments |
| 3 | Thermograph |
| 4-14 | Reserved |
| 15 | Missing value |

0 02 052

- Geostationary imager satellite channels used

| Bit No. | Channel | Central wavelength (micrometer) |
| --- | --- | --- |
| 1 | 1 | 0.55 - 0.75 |
| 2 | 2 | 3.9 |
| 3 | 3 | 6.7 |
| 4 | 4 | 10.7 |
| 5 | 5 | 12.0 |
| All 6 | Missing value |  |

Note:	Beginning with the first bit position (high order bit), if the bit position is set to one, then the channel is used.  If the bit position is set to zero, then the channel is not used.

0 02 053

- GOES-I/M brightness temperature characteristics

| Code figure |  |
| --- | --- |
| 0 | Observed brightness temperature |
| 1 | Brightness temperature with bias correction applied |
| 2 | Brightness temperature calculated from first guess |
| 3 | Brightness temperature calculated from sounding |
| 4-14 | Reserved |
| 15 | Missing value |

0 02 054

- IGOES-I/M soundings parameter characteristics

| Code figure |  |
| --- | --- |
| 0 | Parameter derived using observed sounder brightness temperatures |
| 1 | Parameter derived using observed imager brightness temperatures |
| 2 | Parameter derived using first guess information |
| 3 | Parameter derived NMC analysis information |
| 4 | Parameter derived using radiosonde information |
| 5-14 | Reserved |
| 15 | Missing value |

0 02 055

- Geostationary soundings statistical parameters

| Code figure |  |
| --- | --- |
| 0 | Statistics generated comparing retrieval versus radiosonde |
| 1 | Statistics generated comparing retrieval versus first guess |
| 2 | Statistics generated comparing radiosonde versus first guess |
| 3 | Statistics generated comparing observed versus retrieval |
| 4 | Statistics generated comparing observed versus first guess |
| 5 | Statistics generated comparing radiosonde versus imager |
| 6 | Statistics generated comparing radiosonde versus sounder |
| 7 | Statistics generated for radiosonde |
| 8 | Statistics generated for first guess |
| 9-14 | Reserved |
| 15 | Missing value |

0 02 056

- Geostationary soundings accuracy statistics

| Code figure |  |
| --- | --- |
| 0 | Sums of differences |
| 1 | Sums of squared differences |
| 2 | Simple size |
| 3 | Minimum difference |
| 4 | Maximum difference |
| 5-14 | Reserved |
| 15 | Missing value |

0 02 057

- Origin of first guess information for GOES-I/M soundings

| Code figure |  |
| --- | --- |
| 0 | Nested Grid Model (NGM) |
| 1 | Aviation Model (AVN) |
| 2 | Medium Range Forecast (MRF) Model |
| 3 | Global Data Assimilation System (GDAS) Forecast Model |
| 4 | Prior soundings (within 3 hours o current time) |
| 5 | Climatology |
| 6-14 | Reserved |
| 15 | Missing value |

0 02 058

- Valid times of first guess information of GOES-I/M soundings

| Code figure |  |
| --- | --- |
| 0 | 12 hour and 18 hour |
| 1 | 18 hour and 24 hour |
| 2 | 6 hour and 12 hour |
| 3 | Greater than 24 hours |
| 4-14 | Reserved |
| 15 | Missing value |

0 02 059

- Origin of analysis information for GOES-I/M soundings

| Code figure |  |
| --- | --- |
| 0 | NCEP Nested Grid Model (NGM) Analysis |
| 1 | NCEP Aviation Model (AVN) Analysis |
| 2 | NCEP Medium Range Forecast (MRF) Model Analysis |
| 3 | NCEP Global Data Assimilation System (GDAS) Forecast Model Analysis |
| 4-14 | Reserved |
| 15 | Missing value |

0 02 060

- Origin of surface information for GOES-I/M soundings

| Code figure |  |
| --- | --- |
| 0 | Current surface hourly reports |
| 1 | Current ship reports |
| 2 | Current buoy reports |
| 3 | One hour old surface hourly reports |
| 4 | One hour old ship reports |
| 5 | One hour old buoy reports |
| 6-14 | Reserved |
| 15 | Missing value |

0 02 061

- Aircraft navigational system

| Code figure |  |
| --- | --- |
| 0 | Inertial navigation system |
| 1 | OMEGA |
| 2-6 | Reserved |
| 7 | Missing value |

0 02 062

- Type of aircraft data relay system

| Code figure |  |
| --- | --- |
| 0 | ASDAR |
| 1 | ASDAR (ACARS also available but not operative) |
| 2 | ASDAR (ACARS also available and operative) |
| 3 | ACARS |
| 4 | ACARS (ASDAR also available but not operative) |
| 5 | ACARS (ASDAR also available and operative) |
| 6-14 | Reserved |
| 15 | Missing value |

0 02 064

- Aircraft roll angle angle quality

| Code figure |  |
| --- | --- |
| 0 | Good |
| 1 | Bad |
| 2 | Reserved |
| 3 | Missing value |

- Note:	Bad is currently defined as a roll angle >5 degrees from vertical.

0 02 066

### Radiosonde ground receiving system

| Code figure |  |
| --- | --- |
| 0 | TRS 2000 |
| 1 | IMS 1500C |
| 2-61 | Reserved |
| 62 | Other |
| 63 | Missing value |

0 02 070

- Original specification of latitude/longitude

| Code figure |  |
| --- | --- |
| 0 | Actual location in seconds |
| 1 | Actual location in minutes |
| 2 | Actual location in degrees |
| 3 | Actual location in decidegrees |
| 4 | Actual location in centidegrees |
| 5 | Referenced to checkpoint in seconds |
| 6 | Referenced to checkpoint in minutes |
| 7 | Referenced to checkpoint in degrees |
| 8 | Referenced to checkpoint in decidegrees |
| 9 | Referenced to checkpoint in centidegrees |
| 10 | Actual Location in tenths of a minute |
| 11 | Referenced to checkpoint in tenths of a minute |
| 12-14 | Reserved |
| 15 | Missing value |

0 02 080

## Balloon manufacturer

| Code figure |  |
| --- | --- |
| 0 | Kaysam |
| 1 | Totex |
| 2 | KKS |
| 3-61 | Reserved |
| 62 | Other |
| 63 | Missing value |

0 02 081

## Type of balloon

| Code figure |  |
| --- | --- |
| 0 | GP26 |
| 1 | GP28 |
| 2 | GP30 |
| 3 | HM26 |
| 4 | HM28 |
| 5 | HM30 |
| 6 | SV16 |
| 7-29 | Reserved |
| 30 | Other |
| 31 | Missing value |

0 02 083

## Type of balloon shelter

| Code figure |  |
| --- | --- |
| 0 | High bay |
| 1 | Low bay |
| 2 | Balloon Inflation Launch System (BILS) |
| 3 | Roof-top BILS |
| 4-13 | Reserved |
| 14 | Other |
| 15 | Missing value |

0 02 084

### Type of gas used in balloon

| Code figure |  |
| --- | --- |
| 0 | Hydrogen |
| 1 | Helium |
| 2 | Natural Gas |
| 3-13 | Reserved |
| 14 | Other |
| 15 | Missing value |

0 02 095

## Type of pressure sensor

| Code figure |  |
| --- | --- |
| 0 | Capacitance aneroid |
| 1 | Derived from GPS |
| 2 | Resistive strain gauge |
| 3-29 | Reserved |
| 30 | Other |
| 31 | Missing value |

0 02 096

## Type of temperature sensor

| Code figure |  |
| --- | --- |
| 0 | Rod thermistor |
| 1 | Bead thermistor |
| 2 | Capacitance bead |
| 3-29 | Reserved |
| 30 | Other |
| 31 | Missing value |

0 02 097

## Type of humidity sensor

| Code figure |  |
| --- | --- |
| 0 | VIZ Mark II Carbon Hygristor |
| 1 | VIZ B2 Hygristor |
| 2 | Vaisala A-Humicap |
| 3 | Vaisala H-Humicap |
| 4 | Capacitance sensor |
| 5 | Vaisala RS90 |
| 6 | Sippican Mark IIA Carbon Hygristor |
| 7-29 | Reserved |
| 30 | Other |
| 31 | Missing value |

0 02 101

- Type of antenna

| Code figure |  |
| --- | --- |
| 0 | Centre front-fed paraboloid |
| 1 | Offset front-fed paraboloid |
| 2 | Centre Cassegrain paraboloid |
| 3 | Offset Cassegrain paraboloid |
| 4 | Planar array |
| 5 | Coaxial-collinear array |
| 6 | Yagi elements array |
| 7 | Microstrip |
| 8-13 | Reserved |
| 14 | Other |
| 15 | Missing value |

0 02 103

- Radome

| Bit No. |  |
| --- | --- |
| 1 | Radar antenna is protected by a radome |
| All 2 | Missing value |

0 02 104

- Antenna polarization

| Code figure |  |
| --- | --- |
| 0 | Horizontal polarization |
| 1 | Vertical polarization |
| 2 | Right circular polarization |
| 3 | Left circular polarization |
| 4 | Horizontal and vertical polarization |
| 5 | Right and left circular polarization |
| 6-14 | Reserved |
| 15 | Missing value |

0 02 115

## Type of surface observing equipment

| Code figure |  |
| --- | --- |
| 0 | PDB |
| 1 | RSOIS |
| 2 | ASOS |
| 3 | Psychrometer |
| 4 | F420 |
| 5-29 | Reserved |
| 30 | Other |
| 31 | Missing value |

0 02 119

## Instrument operations

| Code figure |  |
| --- | --- |
| 0 | Intermediate Frequency Calibration Mode (IF CAL) |
| 1 | Built-In Test Equipment Digital (BITE DGT) |
| 2 | Built-In test Equipment Radio Frequency (BITE RF) |
| 3 | Preset tracking ( PSET TRK) |
| 4 | Preset LOOP OUT |
| 5 | ACQUISITION |
| 6 | TRACKING |
| 7 | Missing value |

0 02 131

- Sensitivity time control (STC)

| Bit No. |  |
| --- | --- |
| 1 | STC operational |
| All 2 | Missing values |

0 02 143

- Ozone instrument type

| Code Figure |  |
| --- | --- |
| 0 | Reserved |
| 1 | Brewer spectrophotometer |
| 2 | Caver Teichert |
| 3 | Dobson |
| 4 | Dobson (Japan) |
| 5 | Ehmet |
| 6 | Fecker telescope |
| 7 | Hoelper |
| 8 | Jodmeter |
| 9 | Filter Ozonometer M-83 |
| 10 | Mast |
| 11 | Oxford |
| 12 | Paetzold |
| 13 | Regener |
| 14 | Reserved for future use |
| 15 | Vassy filter Ozonometer |
| 16 | Carbon iodide |
| 17 | Surface ozone bubler |
| 18 | Filter Ozonometer M-124 |
| 19 | ECC sonde |
| 20-126 | Reserved |
| 127 | Missing value |

0 02 144

- Light source type of Brewer spectrophotometer

| Code figure |  |
| --- | --- |
| 0 | Direct Sun |
| 1 | Direct Sun, attenuator #1 |
| 2 | Direct Sun, attenuator #2 |
| 3 | Focussed Moon |
| 4 | Focussed Sun |
| 5 | Focussed Sun corrected with adjacent sky measurements |
| 6 | Zenith Sky |
| 7-14 | Reserved |
| 15 | Missing value |

Note:	Entries 1 and 2 should not be used.

0 02 145

- Wave length setting for Dobson instruments

| Code figure |  |
| --- | --- |
| 0 | Wavelengths AD ordinary setting |
| 1 | Wavelengths BD ordinary setting |
| 2 | Wavelengths CD ordinary setting |
| 3 | Wavelengths CC' ordinary setting |
| 4 | Wavelengths AD focussed image |
| 5 | Wavelengths BD focussed image |
| 6 | Wavelengths CD focussed image |
| 7 | Wavelengths CC' focussed image |
| 8-14 | Reserved |
| 15 | Missing value |

0 02 146

- Source condition for Dobson instruments

| Code figure |  |
| --- | --- |
| 0 | on direct sun |
| 1 | on direct moon |
| 2 | on blue zenith sky |
| 3 | on zenith cloud (uniform stratified layer of small opacity) |
| 4 | on zenith cloud(uniform or moderately variable layer of medium opacity) |
| 5 | on zenith cloud (uniform or moderately variable layer of large opacity) |
| 6 | on zenith cloud (highly variable opacity, with or without precipitation) |
| 7 | on zenith cloud (fog) |
| 8 | On zenith haze |
| 9 | On direct sun through thin cloud, fog or haze |
| 10-14 | reserved |
| 15 | Missing value |

0.02 148

Data collection and/or location system

| Code figure |  |
| --- | --- |
| 0 | Reserved |
| 1 | Argos |
| 2 | GPS |
| 3 | GOES DCP |
| 4 | METEOSAT DCP |
| 5-30 | Reserved |
| 31 | Missing value |

0 02 149

Type of data bouy

| Code figure |  |
| --- | --- |
| 0 | Unspecified drifting buoy |
| 1 | Standard Lagrangian drifter (Global Drifter Programme) |
| 2 | Standard FGGE type drifting buoy (non-Lagrangian meteorological drifting buoy) |
| 3 | Wind measuring FGGE type drifting buoy (non-Lagrangian meteorological drifting buoy) |
| 4 | Ice float |
| 5-7 | Reserved |
| 8 | Unspecified sub-surface float |
| 9 | SOFAR |
| 10 | ALACE |
| 11 | MARVOR |
| 12 | RAFOS |
| 13-15 | Reserved |
| 16 | Unspecified moored buoy |
| 17 | Nomad |
| 18 | 3-metre discus |
| 19 | 10-12-metre discus |
| 20 | ODAS 30 series |
| 21 | ATLAS (e.g. TAO area) |
| 22 | TRITON buoy |
| 23 | Reserved |
| 24 | Omnidirectional waverider |
| 25 | Directional waverider |
| 26 | Sub-surface ARGO float |
| 27-62 | Reserved |
| 63 | Missing value |

0 02 150

- TOVS/ATOVS/AVHRR instrumentation channel number

| code figure |  | Code figure |  |
| --- | --- | --- | --- |
| 0 | Reserved | 28 | AMSU-A 1 |
| 1 | HIRS 1 | 29 | AMSU-A 2 |
| 2 | HIRS 2 | 30 | AMSU-A 3 |
| 3 | HIRS 3 | 31 | AMSU-A 4 |
| 4 | HIRS 4 | 32 | AMSU-A 5 |
| 5 | HIRS 5 | 33 | AMSU-A 6 |
| 6 | HIRS 6 | 34 | AMSU-A 7 |
| 7 | HIRS 7 | 35 | AMSU-A 8 |
| 8 | HIRS 8 | 36 | AMSU-A 9 |
| 9 | HIRS 9 | 37 | AMSU-A 10 |
| 10 | HIRS 10 | 38 | AMSU-A 11 |
| 11 | HIRS 11 | 39 | AMSU-A 12 |
| 12 | HIRS 12 | 40 | AMSU-A 13 |
| 13 | HIRS 13 | 41 | AMSU-A 14 |
| 14 | HIRS 14 | 42 | AMSU-A 15 |
| 15 | HIRS 15 | 43 | AMSU-B 1 |
| 16 | HIRS 16 | 44 | AMSU-B 2 |
| 17 | HIRS 17 | 45 | AMSU-B 3 |
| 18 | HIRS 18 | 46 | AMSU-B 4 |
| 19 | HIRS 19 | 47 | AMSU-B 5 |
| 20 | HIRS 20 | 48 | AVHRR 1 |
| 21 | MSU 1 | 49 | AVHRR 2 |
| 22 | MSU 2 | 50 | AVHRR 3a |
| 23 | MSU 3 | 51 | AVHRR 3b |
| 24 | MSU 4 | 52 | AVHRR 4 |
| 25 | SSU 1 | 53 | AVHRR 5 |
| 26 | SSU 2 | 54-62 | Reserved |
| 27 | SSU 3 | 63 | Missing value |

0 02 151

- Radiometer identifier

| Code figure |  |
| --- | --- |
| 0 | HIRS |
| 1 | MSU |
| 2 | SSU |
| 3 | AMSU-A1-1 |
| 4 | AMSU-A1-2 |
| 5 | AMSU-A2 |
| 6 | AMSU-B |
| 7 | AVHRR |
| 8-2046 | Reserved |
| 2047 | Missing value |

0 02 152

- Satellite instrument data used in processing

| Bit No. |  |
| --- | --- |
| 1 | High-resolution infrared sounder(HIRS) |
| 2 | Microwave sounding unit (MSU) |
| 3 | Stratospheric sounding unit (SSU) |
| 4 | AMI wind mode |
| 5 | AMI wave mode |
| 6 | AMI image mode |
| 7 | RADAR altimeter |
| 8 | ATSR |
| 9 | Geostationary Imager |
| 10 | Geostationary Sounder |
| 11 | Geostationary Earth radiation (GERB) |
| 12-30 | Reserved |
| All 31 | Missing value |

0 02 158

- RA – 2   Instruments

| Bit No. |  |
| --- | --- |
| 1 | Mismatch in RED VEC HPA |
| 2 | Mismatch in RED VEC RFSS |
| 3 | PTR calibration band 320 MHz (Ku) |
| 4 | PTR calibration band 80 MHz (Ku) |
| 5 | PTR calibration band 20 MHz (Ku) |
| 6 | PTR calibration band 160 MHz (S) |
| 7 | Ku flight calibration parameters available |
| 8 | S flight calibration parameters available |
| All 9 | Missing value |

Note:	PTR = Pulse Target Response

HPA = High Power Amplifier

RFSS = Radio Frequency Sub-System

RED = Redundancy

0 02 159

- MWR  Instruments

| Bit No. |  |
| --- | --- |
| 1 | Temperature inconsistency |
| 2 | Data is missing |
| 3 | Redundancy channel |
| 4 | Power bus protection |
| 5 | Overvoltage/Overload protection |
| 6 | Reserved |
| 7 | Reserved |
| All 8 | Missing value |

Note:	MWR - Microwave radiometer

0 02 163

- Height assignment method

| Code figure |  |
| --- | --- |
| 0 | Auto editor |
| 1 | IRW height assignment |
| 2 | WV height assignment |
| 3 | H2O intercept height assignment |
| 4 | CO2 slicing height assignment |
| 5 | Low pixel max gradient |
| 6 | Higher pixel max gradient |
| 7 | Primary height assignment |
| 8 | Layer thickness assignment |
| 9 | Cumulative contribution function -10 percent height |
| 10 | Cumulative contribution function -50 percent height |
| 11 | Cumulative contribution function -90 percent height |
| 12 | Cumulative contribution function - height of maximum gradient |
| 13 | Reserved |
| 14 | Composite height assignment |
| 15 | Missing value |

0 02 164

- Tracer correlation method

| Code figure |  |
| --- | --- |
| 0 | LP - Norms least square minimum |
| 1 | EN - Euclidean norm with radiance correlation |
| 2 | CC - Cross correlation |
| 3-6 | Reserved |
| 7 | Missing value |

0 02 166

- Radiance type

| Code figure |  |
| --- | --- |
| 0 | Type not defined |
| 1 | Automated statistical regression |
| 2 | Clear path |
| 3 | Partly cloudy path |
| 4 | Cloudy path |
| 5-14 | Reserved |
| 15 | Missing value |

0 02 167

- Radiance computational method

| Code figure |  |
| --- | --- |
| 0 | Method not defined |
| 1 | 1b raw radiance |
| 2 | processed radiance |
| 3-14 | Reserved |
| 15 | Missing value |

0 02 169

Anemometer type

| Code figure |  |
| --- | --- |
| 0 | Cup rotor |
| 1 | Propeller rotor |
| 2 | Wind Observation Through Ambient Noise (WOTAN) |
| 3-14 | Reserved |
| 15 | Missing value |

0 02 172

Product type for retrieved atmospheric gases

| Code figure |  |
| --- | --- |
| 0 | Reserved |
| 1 | Retrieval from a nadir sounding |
| 2 | Retrieval from a limb sounding |
| 3-254 | Reserved |
| 255 | Missing value |

0 02 175

Method of precipitation measurement

| Code figure |  |
| --- | --- |
| 0 | Manual measurement |
| 1 | Tipping bucket method |
| 2 | Weighing method |
| 3 | Optical method |
| 4 | Pressure method |
| 5 | Float method |
| 6 | Drop counter method |
| 7-13 | Reserved |
| 14 | Others |
| 15 | Missing value |

0 02 176

Method of state of ground measurement

| Code figure |  |
| --- | --- |
| 0 | Manual observation |
| 1 | Video camera method |
| 2 | Infra-red method |
| 3 | Laser method |
| 4-13 | Reserved |
| 14 | Others |
| 15 | Missing value |

0 02 177

Method of snow depth measurement

| Code figure |  |
| --- | --- |
| 0 | Manual observation |
| 1 | Ultrasonic method |
| 2 | Video camera method |
| 3-13 | Reserved |
| 14 | Others |
| 15 | Missing value |

0 02 178

Method of liquid content measurement of precipitation

| Code figure |  |
| --- | --- |
| 0 | Manual observation |
| 1 | Optical method |
| 2 | Capacitive method |
| 3-13 | Reserved |
| 14 | Others |
| 15 | Missing value |

0 02 179

Type of sky condition algorithm

| Code figure |  |
| --- | --- |
| 0 | Manual observation |
| 1 | VAISALA algorithm |
| 2 | ASOS (FAA) algorithm |
| 3 | AWOS (Canada) algorithm |
| 4-13 | Reserved |
| 14 | Others |
| 15 | Missing value |

0 02 180

Main present weather detecting system

| Code Figure |  |
| --- | --- |
| 0 | Manual observation |
| 1 | Optical scatter system combined with precipitation occurrence sensing system |
| 2 | Forward and/or back-scatter system of visible light |
| 3 | Forward and/or back-scatter system of infrared light |
| 4 | Infrared light emitting diode (IRED) system |
| 5 | Doppler radar system |
| 6-13 | Reserved |
| 14 | Others |
| 15 | Missing value |

0 02 181

Supplementary present weather sensor

| Bit No. |  |
| --- | --- |
| 1 | Rain detector |
| 2 | Freezing rain sensor |
| 3 | Ice detection sensor |
| 4 | Hail and ice pellet sensor |
| 5-19 | Reserved |
| 20 | Others |
| All 21 | Missing value |

0 02 182

Visibility measurement system

| Code figure |  |
| --- | --- |
| 0 | Manual measurement |
| 1 | Transmissometer system (base ≥ 25 m) |
| 2 | Transmissometer system (base < 25 m) |
| 3 | Forward scatter system |
| 4 | Back scatter system |
| 5-13 | Reserved |
| 14 | Others |
| 15 | Missing value |

0 02 183

Cloud detection system

| Code figure |  |
| --- | --- |
| 0 | Manual observation |
| 1 | Ceilometer system |
| 2 | Infrared camera system |
| 3 | Microwave visual camera system |
| 4 | Sky imager system |
| 5 | Video time lapsed camera system |
| 6 | Micro pulse lidar (MPL) system |
| 7-13 | Reserved |
| 14 | Others |
| 15 | Missing value |

0 02 184

Type of lightning detection sensor

| Code figure |  |
| --- | --- |
| 0 | Manual observation |
| 1 | Lightning imaging sensor |
| 2 | Electrical storm identification sensor |
| 3 | Magnetic finder sensor |
| 4 | Lightning strike sensor |
| 5 | Flash counter |
| 6-13 | Reserved |
| 14 | Others |
| 15 | Missing value |

0 02 185

Method of evaporation measurement

| Code Figure |  |
| --- | --- |
| 0 | Manual measurement |
| 1 | Balanced floating method |
| 2 | Pressure method |
| 3 | Ultrasonic method |
| 4 | Hydraulic method |
| 5-13 | Reserved |
| 14 | Others |
| 15 | Missing value |

0 02 186

Capability to detect precipitation phenomena

| Bit No. |  |
| --- | --- |
| 1 | Precipitation-unknown type |
| 2 | Liquid precipitation not freezing |
| 3 | Liquid freezing precipitation |
| 4 | Drizzle |
| 5 | Rain |
| 6 | Solid precipitation |
| 7 | Snow |
| 8 | Snow grains |
| 9 | Snow pellets |
| 10 | Ice pellets |
| 11 | Ice crystals |
| 12 | Diamond dust |
| 13 | Small hail |
| 14 | Hail |
| 15 | Glaze |
| 16 | Rime |
| 17 | Soft rime |
| 18 | Hard rime |
| 19 | Clear ice |
| 20 | Wet snow |
| 21 | Hoar frost |
| 22 | Dew |
| 23 | White dew |
| 24-29 | Reserved |
| All 30 | Missing value |

0 02 187

# Capability to detect other weather phenomena

| Bit No. |  |
| --- | --- |
| 1 | Dust/sand whirl |
| 2 | Squalls |
| 3 | Sand storm |
| 4 | Dust storm |
| 5 | Lightning - cloud to surface |
| 6 | Lightning - cloud to cloud |
| 7 | Lightning - distant |
| 8 | Thunderstorm |
| 9 | Funnel Cloud not touching surface |
| 10 | Funnel cloud touching surface |
| 11 | Spray |
| 12-17 | Reserved |
| All 18 | Missing value |

0 02 188

Capability to detect obscuration

| Bit No. |  |
| --- | --- |
| 1 | Fog |
| 2 | Ice fog |
| 3 | Steam fog |
| 4-6 | Reserved |
| 7 | Mist |
| 8 | Haze |
| 9 | Smoke |
| 10 | Volcanic ash |
| 11 | Dust |
| 12 | Sand |
| 13 | Snow |
| 14-20 | Reserved |
| All 21 | Missing value |

0 02 189

Capability to discriminate lightning strikes

| Bit No. |  |
| --- | --- |
| 1 | Manual observation |
| 2 | All lightning strikes without discrimination |
| 3 | Lightning strikes cloud to ground only |
| 4 | All lightning strikes with discrimination between cloud to ground and cloud to cloud |
| 5-11 | Reserved |
| All 12 | Missing value |

0 04 059

Times of observation used to compute the reported mean values

| Bit No. |  |
| --- | --- |
| 1 | 00 UTC |
| 2 | 06 UTC |
| 3 | 12 UTC |
| 4 | 18 UTC |
| 5 | Other hours |
| All 6 | Missing value |

0 08 001

- Vertical sounding significance

| Bit No. |  |
| --- | --- |
| 1 | Surface |
| 2 | Standard level |
| 3 | Tropopause level |
| 4 | Maximum wind level |
| 5 | Significant level, temperature and/or relative humidity |
| 6 | Significant level, wind |
| All 7 | Missing value |

0 08 002

- Vertical significance (surface observations)

| Code figure |  |
| --- | --- |
| 0 | Observing rules for base of lowest cloud and cloud types of FM 12SYNOP and FM 13 SHIP apply |
| 1 | First non - Cb significant layer |
| 2 | Second non - Cb significant layer |
| 3 | Third non - Cb significant layer |
| 4 | Cumulonimbus layer |
| 5 | Ceiling |
| 6 | Clouds not detected below the following height(s) |
| 7 | Low cloud |
| 8 | Middle cloud |
| 9 | High cloud |
| 10 | Cloud layer with base below the station level and top above the station level |
| 11 | Cloud layer with base and top below the station level |
| 12-62 | Reserved |
| 63 | Missing value |

0 08 003

- Vertical significance (satellite observations)

| Code figure |  |
| --- | --- |
| 0 | Surface |
| 1 | Base of Satellite sounding |
| 2 | Cloud top |
| 3 | Tropopause |
| 4 | Precipitable water |
| 5 | Sounding Radiances |
| 6 | Mean Temperatures |
| 7 | Ozone |
| 8-62 | Reserved |
| 63 | Missing value |

0 08 004

- Phase of aircraft flight

| Code figure |  |
| --- | --- |
| 0-1 | Reserved |
| 2 | Unsteady (UNS) |
| 3 | Level flight, routine observation (LVR) |
| 4 | Level flight, highest wind encountered (LVW) |
| 5 | Ascending (ASC) |
| 6 | Descending (DES) |
| 7 | Missing value |

0 08 005

- Meteorological attribute significance

| Code figure |  |
| --- | --- |
| 0 | Reserved |
| 1 | Storm center |
| 2 | Outer limit or edge of storm |
| 3 | Location of maximun wind |
| 4 | Location of the storm in the perturbed analysis |
| 5 | Location of the storm in the analysis |
| 6-14 | Reserved |
| 15 | Missing value |

0 08 006

Ozone vertical sounding significance

| Bit No. |  |
| --- | --- |
| 1 | Surface |
| 2 | Standard level |
| 3 | Tropopause level |
| 4 | Prominent maximum level |
| 5 | Prominent minimum level |
| 6 | Minimum pressure level |
| 7 | Reserved |
| 8 | Level of undetermined significance |
| All 9 | Missing value |

0 08 007

- Dimensional significance

| Code figure |  |
| --- | --- |
| 0 | Point |
| 1 | Line |
| 2 | Area |
| 3 | Volume |
| 4-14 | Reserved |
| 15 | Missing value |

Note:	A consecutive sequence of 2 or more of location coordinates, such as latitude and longitude pairs, defines a line or polygon.  Points shall be joined in the order given in the message.  Any area described will fall left of the drawn boundary in the direction established by the order of the points given in the message.  This definition is for simple non-intersecting polygons without holes.

0 08 008

Radiation vertical sounding significance

| Bit No. |  |
| --- | --- |
| 1 | Surface |
| 2 | Standard level |
| 3 | Tropopause level |
| 4 | Level of beta radiation maximum |
| 5 | Level of gamma radiation maximum |
| 6 | Minimum pressure level |
| 7 | Reserved |
| 8 | Level of undetermined significance |
| All 9 | Missing value |

0 08 009

- Detailed phase of aircraft flight

| Code figure |  |
| --- | --- |
| 0 | Level flight, routine observation, unsteady |
| 1 | Level flight, highest wind encountered, unsteady |
| 2 | Unsteady (UNS) |
| 3 | Level flight, routine observation (LVR) |
| 4 | Level flight, highest wind encountered (LVW) |
| 5 | Ascending (ASC) |
| 6 | Descending (DES) |
| 7 | Ascending, observation intervals selected by time increments |
| 8 | Ascending, observation intervals selected by time increments, unsteady |
| 9 | Ascending, observation intervals selected by pressure increments |
| 10 | Ascending, observation intervals selected by pressure increments, unsteady |
| 11 | Descending, observation intervals selected by time increments |
| 12 | Descending, observation intervals selected by time increments, unsteady |
| 13 | Descending, observation intervals selected by pressure increments |
| 14 | Descending, observation intervals selected by pressure increments, unsteady |
| 15 | Missing value |

0 08 010

Surface qualifier (for temperature data)

| Code figure |  |
| --- | --- |
| 0 | Reserved |
| 1 | Bare soil |
| 2 | Bare rock |
| 3 | Land grass cover |
| 4 | Water (lake, sea) |
| 5 | Flood water underneath |
| 6 | Snow |
| 7 | Ice |
| 8 | Runway or road |
| 9 | Ship or platform deck in steel |
| 10 | Ship or platform deck in wood |
| 11 | Ship or platform deck partly covered with rubber mat |
| 12-30 | Reserved |
| 31 | Missing value |

0 08 011

- Meteorological Feature

| Code figure |  |
| --- | --- |
| 0 | Quasi-stationary front at the surface |
| 1 | Quasi-stationary front above the surface |
| 2 | Warm front at the surface |
| 3 | Warm front above the surface |
| 4 | Cold front at the surface |
| 5 | Cold front above the surface |
| 6 | Occlusion |
| 7 | Instability line |
| 8 | Intertropical front |
| 9 | Convergence line |
| 10 | Jet stream |
| 11 | Cloud clear |
| 12 | Cloud |
| 13 | Turbulence |
| 14 | Storm |
| 15 | Airframe icing |
| 16 | Phenomenon |
| 17 | Volcano |
| 18 | Atmospherics |
| 19 | Reserved |
| 20 | Special clouds |
| 21 | Thunderstorm (VAL) |
| 22 | Tropical cyclone (VAL) |
| 23 | Mountain Wave (VAL) |
| 24 | Duststorm (VAL) |
| 25 | Sandstorm (VAL) |
| 26-62 | Reserved |
| 63 | Missing value |

VAL = code figure in validation process

0 08 012

- Land/sea qualifier

| Code figure |  |
| --- | --- |
| 0 | Land |
| 1 | Sea |
| 2 | Coastal |
| 3 | Missing value |

0 08 013

- Day/Night qualifier

| Code Figure |  |
| --- | --- |
| 0 | Night |
| 1 | Day |
| 2 | Reserved |
| 3 | Missing value |

0 08 014

- Qualifier for runway visual range

| Code figures |  |  |
| --- | --- | --- |
| 0 | 10-minute mean value | - normal value |
| 1 | 10-minute mean value | - above the upper limit for assessments of RVR (P) |
| 2 | 10-minute mean value | - below the lower limit for assessments of RVR (M) |
| 3 | one-minute minimum value | - normal value |
| 4 | one-minute minimum value | - above the upper limit for assessments of RVR (P) |
| 5 | one-minute minimum value | - below the lower limit for assessments of RVR (M) |
| 6 | one-minute maximum value | - normal value |
| 7 | one-minute maximum value | - above the upper limit for assessments of RVR (P) |
| 8 | one-minute maximum value | - below the lower limit for assessments of RVR (M) |
| 9-14 | Reserved |  |
| 15 | Missing value |  |

0 08 016

- Change qualifiers of a trend-type forecast or an aerodrome forecast

| Code figures |  |
| --- | --- |
| 0 | NOSIG |
| 1 | BECMG |
| 2 | TEMPO |
| 3 | FM |
| 4-6 | Reserved |
| 7 | Missing value |

0 08 017

- Qualifier of the time when the forecast change is expected

| Code figures |  |
| --- | --- |
| 0 | FM |
| 1 | TL |
| 2 | AT |
| 3 | Missing value |

0 08 018

SEAWINDS land/ice surface type

| Bit No. |  |
| --- | --- |
| 1 | Land is present |
| 2 | Surface ice map indicates ice is present |
| 3-10 | Reserved |
| 11 | Ice map data not available |
| 12 | Attenuation map data not available |
| 13-16 | Reserved |
| All 17 | Missing value |

0 08 021

- Time significance

| Code figure |  |
| --- | --- |
| 0 | Reserved |
| 1 | Time series |
| 2 | Time averaged |
| 3 | Accumulated |
| 4 | Forecast |
| 5 | Forecast time series |
| 6 | Forecast time averaged |
| 7 | Forecast accumulated |
| 8 | Ensemble mean |
| 9 | Ensemble mean time series |
| 10 | Ensemble mean time averaged |
| 11 | Ensemble mean accumulated |
| 12 | Ensemble mean forecast |
| 13 | Ensemble mean forecast time series |
| 14 | Ensemble mean forecast time averaged |
| 15 | Ensemble mean forecast accumulated |
| 16 | Analysis |
| 17 | Start of phenomenon |
| 18 | Radiosonde launch time |
| 19 | Start of orbit |
| 20 | End of orbit |
| 21 | Time of ascending node |
| 22 | Time of occurrence of wind shift |
| 23 | Monitoring period |
| 24 | Agreed time limit for report reception |
| 25 | Nominal reporting time |
| 26 | Time of last known position |
| 27 | First guess |
| 28 | Start of scan |
| 29 | End of scan |
| 30 | Reserved |
| 31 | Missing value |

Notes:

(1)	"Time averaged" indicates that values are continuously averaged over a period of time.

(2)	"Ensemble mean" indicates that a number of distinct values corresponding to a set of time locations are averaged.

(3)	Time significance must be qualified by appropriate time periods being specified.

0 08 023

- First Order Statistics

| Code Figure |  |
| --- | --- |
| 0 | Reserved |
| 1 | Reserved |
| 2 | Maximum value |
| 3 | Minimum value |
| 4 | Mean value |
| 5 | Median value |
| 6 | Modal value |
| 7 | Mean absolute error |
| 8 | Reserved |
| 9 | Best estimate of standard deviation (N-1) |
| 10 | Standard deviation (N) |
| 11 | Harmonic mean |
| 12 | Root-mean-square vector error |
| 13 | Root-mean-square |
| 14-31 | Reserved |
| 32 | Vector mean |
| 33-62 | Reserved for local use |
| 63 | Missing value |

NOTE:	All first order statistics are in the units defined by the original data descriptors.

0 08 024

- Difference Statistics

| Code Figure |  |
| --- | --- |
| 0 | Reserved |
| 1 | Reserved |
| 2 | Observed minus maximum |
| 3 | Observed minus minimum |
| 4 | Observed minus mean |
| 5 | Observed minus median |
| 6 | Observed minus mode |
| 7-10 | Reserved |
| 11 | Observed minus climatology (anomaly) |
| 12 | Observed minus analyzed value |
| 13 | Observed minus initialized analysed value |
| 14 | Observed minus forecast value |
| 15-20 | Reserved |
| 21 | Observed minus interpolated value |
| 22 | Observed minus hydrostatically calculated value |
| 23-31 | Reserved |
| 32-62 | Reserved for local use |
| 63 | Missing value |

Notes:

- Difference statistics are difference values; they have dimensions the same as the corresponding reported values with respect to units, but assume a range centred on zero (e.g., the difference between reported and analysed values, the difference between reported and forecast values, etc.).

- Where observed minus forecast values are represented, the period of the forecast shall be indicated by an appropriate descriptor from class 4.

0 08 025

- Time difference qualifier

| Code Figure |  |
| --- | --- |
| 0 | Universal Time Coordinated (UTC) minus Local Standard Time (LST) |
| 1 | Local Standard Time |
| 2 | Universal Time Coordinated (UTC) minus Satellite clock |
| 3-4 | Reserved |
| 5 | Time difference from edge of processing segment |
| 6-14 | Reserved |
| 15 | Missing value |

0 08 029

## Remotely sensed surface type

| Code Figure |  |
| --- | --- |
| 0 | Open ocean or semi-enclosed sea |
| 1 | Enclosed sea or lake |
| 2 | Continental ice |
| 3 | Land |
| 4-254 | Reserved |
| 255 | Missing value |

0 08 033

Method of derivation of percentage confidence

| Code Figure |  |
| --- | --- |
| 0 | Reserved |
| 1 | Percentage confidence calculated using cloud fraction |
| 2 | Percentage confidence calculated using standard deviation of temperature |
| 3 | Percentage confidence calculated using probability of cloud contamination |
| 4 | Percentage confidence calculated using normality of distribution |
| 5-126 | Reserved |
| 127 | Missing value |

0 08 035

Type of monitoring exercise

| Code figure |  |
| --- | --- |
| 0 | Global |
| 1 | Regional |
| 2 | National |
| 3 | Special |
| 4 | Bilateral |
| 5 | Reserved |
| 6 | Reserved |
| 7 | Missing value |

0 08 036

Type of centre or station performing monitoring

| Code figure |  |
| --- | --- |
| 0 | WMO Secretariat |
| 1 | WMO |
| 2 | RSMC |
| 3 | NMC |
| 4 | RTH |
| 5 | Observing site |
| 6 | Other |
| 7 | Missing value |

0 08 040

Flight level significance

| Code figure |  |
| --- | --- |
| 0 | High resolution data sample |
| 1 | Within 20 hPa of surface |
| 2 | Pressure less than 10 hPa (i.e., 9, 8, 7, etc.) when no other reason applies |
| 3 | Base pressure level for stability index |
| 4 | Begin doubtful temperature, height data |
| 5 | Begin missing data (all elements) |
| 6 | Begin missing RH data |
| 7 | Begin missing temperature data |
| 8 | Highest level reached before balloon descent because of icing or turbulence |
| 9 | End doubtful temperature, height data |
| 10 | End missing data (all elements) |
| 11 | End missing RH data |
| 12 | End missing temperature data |
| 13 | Zero degrees C crossing(s) for RADAT |
| 14 | Standard pressure level |
| 15 | Operator added level |
| 16 | Operator deleted level |
| 17 | Balloon re-ascended beyond previous highest ascent level |
| 18 | Significant RH level |
| 19 | RH level selection terminated |
| 20 | Surface level |
| 21 | Significant temperature level |
| 22 | Mandatory temperature level |
| 23 | Flight termination level |
| 24 | Tropopause(s) |
| 25 | Aircraft report |
| 26 | Interpolated (generated) level |
| 27 | Mandatory wind level |
| 28 | Significant wind level |
| 29 | Maximum wind level |
| 30 | Incremental wind level (fixed regional) |
| 31 | Incremental height level (generated) |
| 32 | Wind termination level |
| 33 | Pressure 100 to 110 hPa, when no other reason applies |
| 34-39 | Reserved |
| 40 | Significant thermodynamic level (inversion) |
| 41 | Significant RH level ( per NCDC criteria) |
| 42 | Significant temperature level (per NCDC) |
| 43 | Begin missing wind data |
| 44 | End missing wind data |
| 45-59 | Reserved |
| 60 | Level of 80-knot isotach above jet |
| 61 | Level of 80-knot isotach below jet |
| 62 | Other |
| 63 | Missing value |

0 08 041

Data significance

| Code figure |  |
| --- | --- |
| 0 | Parent site |
| 1 | Observation site |
| 2 | Balloon manufacture date |
| 3 | Balloon launch point |
| 4 | Surface observation |
| 5 | Surface observation displacement from launch point |
| 6 | Flight level observation |
| 7 | Flight level termination point |
| 8-30 | Reserved |
| 31 | Missing value |

#### 0 08 042

#### Extended vertical sounding significance

| Bit No. |  |
| --- | --- |
| 1 | Surface |
| 2 | Standard level |
| 3 | Tropopause level |
| 4 | Maximum wind level |
| 5 | Significant temperature level |
| 6 | Significant humidity level |
| 7 | Significant wind level |
| 8 | Beginning of missing temperature data |
| 9 | End of missing temperature data |
| 10 | Beginning of missing humidity data |
| 11 | End of missing humidity data |
| 12 | Beginning of missing wind data |
| 13 | End of missing wind data |
| 14-17 | Reserved |
| All 18 | Missing value |

0 08 050

#### Qualifier for number of missing values in calculation of statistic

| Code figure |  |
| --- | --- |
| 0 | Reserved |
| 1 | Pressure |
| 2 | Temperature |
| 3 | Extreme temperature |
| 4 | Vapour pressure |
| 5 | Precipitation |
| 6 | Sunshine duration |
| 7 | Maximum temperature |
| 8 | Minimum temperature |
| 9 | Wind |
| 10-14 | Reserved |
| 15 | Missing value |

0 08 051

- Qualifier for number of missing values in calculation of statistic

| Code figure |  |
| --- | --- |
| 1 | Pressure |
| 2 | Temperature |
| 3 | Extreme temperature |
| 4 | Vapour pressure |
| 5 | Precipitation |
| 6 | Sunshine duration |
| 7 | Missing value |

0 08 052

- Condition for which number of days of occurrence follows

| Code figure |  |
| --- | --- |
| 0 | Mean wind speed over a 10-minute period observed or recorded equal to or more than 10 m/s or 20 knots |
| 1 | Mean wind speed over a 10-minute period observed or recorded equal to or more than 20 m/s or 40 knots |
| 2 | Mean wind speed over a 10-minute period observed or recorded equal to or more than 30 m/s or 60 knots |
| 3 | Maximum temperature less than 273.2 K |
| 4 | Maximum temperature equal to or more than 298.2 K |
| 5 | Maximum temperature equal to or more than 303.2 K |
| 6 | Maximum temperature equal to or more than 308.2 K |
| 7 | Maximum temperature equal to or more than 313.2 K |
| 8 | Minimum temperature less than 273.2 K |
| 9 | Maximum temperature equal to or more than 273.2 K |
| 10 | Precipitation equal to or more than 1.0 kgm-2 |
| 11 | Precipitation equal to or more than 5.0 kgm-2 |
| 12 | Precipitation equal to or more than 10.0 kgm-2 |
| 13 | Precipitation equal to or more than 50.0 kgm-2 |
| 14 | Precipitation equal to or more than 100.0 kgm-2 |
| 15 | Precipitation equal to or more than 150.0 kgm-2 |
| 16 | Snow depth more than 0.00 m |
| 17 | Snow depth more than 0.01 m |
| 18 | Snow depth more than 0.10 m |
| 19 | Snow depth more than 0.50 m |
| 20 | Horizontal visibility less than 50 m |
| 21 | Horizontal visibility less than 100 m |
| 22 | Horizontal visibility less than 1000 m |
| 23 | Hail |
| 24 | Thunderstorm |
| 25-30 | Reserved |
| 31 | Missing value |

0 08 053

- Day of occurrence qualifier

| Code figure |  |
| --- | --- |
| 0 | Value occurred on only one day in the month |
| 1 | Value occurred on more than one day in the month |
| 2 | Reserved |
| 3 | Missing value |

0 08 060

- Sample Scanning Mode Significance

| Code figure |  |
| --- | --- |
| 0 | Reserved |
| 1 | Range |
| 2 | Azimuth |
| 3 | Horizontal |
| 4 | Vertical |
| 5 | North/South |
| 6 | East/West |
| 7-14 | Reserved |
| 15 | Missing value |

0 08 065

- Sun-glint indicator

| Code figure |  |
| --- | --- |
| 0 | No sun-glint |
| 1 | Sun-glint |
| 2 | Reserved |
| 3 | Missing value |

0 08 066

- Semi-transparency indicator

| Code figure |  |
| --- | --- |
| 0 | Opaque |
| 1 | Semi-transparent |
| 2 | Reserved |
| 3 | Missing value |

0 08 070

- TOVS/ATOVS product qualifier

| Code figure |  |
| --- | --- |
| 0 | Reserved |
| 1 | Reserved |
| 2 | Earth located instrument counts, calibration coefficients and housekeeping (level 1b) |
| 3 | Earth located calibrated radiances (level 1c) |
| 4 | Mapped to a common footprint, earth located calibrated radiances (level 1d) |
| 5-14 | Reserved |
| 15 | Missing value |

0 08 072

- Pixel(s) type

| Code figure |  |
| --- | --- |
| 0 | Mixed |
| 1 | Clear |
| 2 | Cloudy |
| 3-6 | Reserved |
| 7 | Missing value |

0 08 074

##### Altimeter echo type

| Code figure |  |
| --- | --- |
| 0 | Open ocean or semi-enclosed sea |
| 1 | Non-ocean like |
| 2 | Reserved |
| 3 | Missing value |

0-08-075

Ascending/Descending Orbit Qualifier

| Code figure |  |
| --- | --- |
| 0 | Ascending orbit |
| 1 | Descending orbit |
| 2 | Reserved |
| 3 | Missing value |

0 08 076

## Type of band

| Code figure |  |
| --- | --- |
| 0 | Ku |
| 1 | C |
| 2-62 | Reserved |
| 63 | Missing value |

0 08 080

- Qualifier for GTSPP quality flag

| Code figure |  |
| --- | --- |
| 0 | Total water pressure profile |
| 1 | Total water temperature profile |
| 2 | Total water salinity profile |
| 3 | Total water conductivity profile |
| 4-62 | Reserved |
| 63 | Missing value |

0 08 081

Type of equipment

| Code figure |  |
| --- | --- |
| 0 | Sensor |
| 1 | Transmitter |
| 2 | Receiver |
| 3 | Observing platform |
| 4-62 | Reserved |
| 63 | Missing value |

0 08 082

Modification of sensor height to another value

| Code figure |  |
| --- | --- |
| 0 | Sensor height is not modified |
| 1 | Sensor height is modified to standard level |
| 2-6 | Reserved |
| 7 | Missing value |

Note: If 0 08 082 = 1, standard level is indicated by the descriptor of class 7, which immediately follows.  It is possible to indicate the real height of the sensor by preceding the descriptor by relevant class 7 descriptor.

0 10 063

- Characteristic of pressure tendency

| Code figure |  |  |
| --- | --- | --- |
| 0 | Increasing, then decreasing; atmospheric pressure the same or higher than 3 hours ago |  |
| 1 | Increasing, then steady; or increasing, then increasing more slowly | Atmospheric pressure now higher than 3 hours ago. |
| 2 | Increasing (steadily or unsteadily) |  |
| 3 | Decreasing or steady, then increasing; or increasing, then increasing more rapidly |  |
| 4 | Steady; atmospheric pressure the same as 3 hours ago |  |
| 5 | Decreasing, then increasing; atmospheric pressure the same or lower than 3 hours ago |  |
| 6 | Decreasing, then steady; or decreasing, then decreasing more slowly | Atmospheric pressure now lower than 3 hours ago. |
| 7 | Decreasing (steadily or unsteadily) |  |
| 8 | Steady or increasing, then decreasing; or decreasing, then decreasing more rapidly |  |
| 9-14 | Reserved |  |
| 15 | Missing value |  |

Notes:

(1)	In reports from automatic stations, code figure 2 shall be used when tendency is positive, 7 when negative, and 4 when the pressure is the same as 3 hours before.

(2)	In reports from tropical stations reporting 24-hour pressure changes, code figure 2 shall be used when tendency is positive, 7 when negative, and 4 when pressure Is the same as 24 hours before.

0 11 031

- Degree of turbulence

| Code figure |  |  |
| --- | --- | --- |
| 0 | Nil | In cloud |
| 1 | Light |  |
| 2 | Moderate |  |
| 3 | Severe |  |
| 4 | Nil | In clear air |
| 5 | Light |  |
| 6 | Moderate |  |
| 7 | Severe |  |
| 8 | Nil | Cloud/clear air not specified |
| 9 | Light |  |
| 10 | Moderate |  |
| 11 | Severe |  |
| 12 | Extreme, In clear air |  |
| 13 | Extreme, In cloud |  |
| 14 | Extreme, cloud/clear air not specified |  |
| 15 | Missing value |  |

0 11 037

Turbulence index

| Code figure | Average Value of Eddy Dissipation Rate (ave) (m2/3 s-1) | Peak Value of Eddy Dissipation Rate (peak) (m2/3 s-1) |
| --- | --- | --- |
| 0 | ave <0.1 | peak <0.1 |
| 1 | ave <0.1 | 0.1 <= peak <0.2 |
| 2 | 0.1 <= ave <0.2 | 0.1 <= peak <0.2 |
| 3 | ave <0.1 | 0.2 <= peak <0.3 |
| 4 | 0.1 <= ave <0.2 | 0.2 <= peak <0.3 |
| 5 | 0.2 <= ave <0.3 | 0.2 <= peak <0.3 |
| 6 | ave <0.1 | 0.3 <= peak <0.4 |
| 7 | 0.1 <= ave <0.2 | 0.3 <= peak <0.4 |
| 8 | 0.2 <= ave <0.3 | 0.3 <= peak <0.4 |
| 9 | 0.3 <= ave <0.4 | 0.3 <= peak <0.4 |
| 10 | ave <0.1 | 0.4 <= peak <0.5 |
| 11 | 0.1 <= ave <0.2 | 0.4 <= peak <0.5 |
| 12 | 0.2 <= ave <0.3 | 0.4 <= peak <0.5 |
| 13 | 0.3 <= ave <0.4 | 0.4 <= peak <0.5 |
| 14 | 0.4 <= ave <0.5 | 0.4 <= peak <0.5 |
| 15 | ave <0.1 | 0.5 <= peak <0.8 |
| 16 | 0.1 <= ave <0.2 | 0.5 <= peak <0.8 |
| 17 | 0.2 <= ave <0.3 | 0.5 <= peak <0.8 |
| 18 | 0.3 <= ave <0.4 | 0.5 <= peak <0.8 |
| 19 | 0.4 <= ave <0.5 | 0.5 <= peak <0.8 |
| 20 | 0.5 <= ave <0.8 | 0.5 <= peak <0.8 |
| 21 | ave <0.1 | 0.8 <= peak |
| 22 | 0.1 <= ave <0.2 | 0.8 <= peak |
| 23 | 0.2 <= ave <0.3 | 0.8 <= peak |
| 24 | 0.3 <= ave <0.4 | 0.8 <= peak |
| 25 | 0.4 <= ave <0.5 | 0.8 <= peak |
| 26 | 0.5 <= ave <0.8 | 0.8 <= peak |
| 27 | 0.8 <= ave | 0.8 <= peak |
| 28 | Nil | Nil |
| 29-62 | Reserved | Reserved |
| 63 | Missing value | Missing value |

0 11 038

Time of Occurrence of Peak Eddy Dissipation Rate

| Code figure | Minutes prior to observation time (min) |
| --- | --- |
| 0 | min < 1 |
| 1 | 1 <= min < 2 |
| 2 | 2 <= min < 3 |
| 3 | 3 <= min < 4 |
| 4 | 4 <= min < 5 |
| 5 | 5 <= min < 6 |
| 6 | 6 <= min < 7 |
| 7 | 7 <= min < 8 |
| 8 | 8 <= min < 9 |
| 9 | 9 <= min < 10 |
| 10 | 10 <= min < 11 |
| 11 | 11 <= min < 12 |
| 12 | 12 <= min < 13 |
| 13 | 13 <= min < 14 |
| 14 | 14 <= min < 15 |
| 15 | No timing information available |
| 16-30 | Reserved |
| 31 | Missing value |

0 11 039

Extended Time of Occurrence of Peak Eddy Dissipation Rate

| Code figure | Minutes prior to observation time (min) |
| --- | --- |
| 0 | min < 1 |
| 1 | 1 <= min < 2 |
| 2 | 2 <= min < 3 |
| 3 | 3 <= min < 4 |
| 4 | 4 <= min < 5 |
| 5 | 5 <= min < 6 |
| 6 | 6 <= min < 7 |
| 7 | 7 <= min < 8 |
| 8 | 8 <= min < 9 |
| 9 | 9 <= min < 10 |
| 10 | 10 <= min < 11 |
| 11 | 11 <= min < 12 |
| 12 | 12 <= min < 13 |
| 13 | 13 <= min < 14 |
| 14 | 14 <= min < 15 |
| 15-59 | As above to 59 <=min < 60 |
| 60 | No timing information available |
| 61-62 | Reserved |
| 63 | Missing value |

0 13 038

- Superadabiatic indicator

| Code figure |  |
| --- | --- |
| 0 | Not superadiabatic |
| 1 | Superadiabatic |
| 2 | Reserved |
| 3 | Missing value |

0 13 039

- Terrain type (ice/snow)

| Code figure |  |
| --- | --- |
| 0 | Sea ice |
| 1 | Snow on land |
| 2-6 | Reserved |
| 7 | Missing value |

0 13 040

###### Surface flag

| Code figure |  |
| --- | --- |
| 0 | Land |
| 1 | Reserved |
| 2 | Near coast |
| 3 | Ice |
| 4 | Possible ice |
| 5 | Ocean |
| 6 | Coast |
| 7-14 | Reserved |
| 7 | Missing value |

0 13 041

- Pasquill-Gifford stability category

| Code figure |  |
| --- | --- |
| 1 | A |
| 2 | A-B |
| 3 | B |
| 4 | B-C |
| 5 | C |
| 6 | D |
| 7 | E |
| 8 | F |
| 9 | G |
| 10-14 | Reserved |
| 15 | Missing value |

0 13 051

- Frequency group, precipitation

| Code figure |  |
| --- | --- |
| 0 | Smaller than any value in the 30-year period |
| 1 | In the first quintile |
| 2 | In the second quintile |
| 3 | In the third quintile |
| 4 | In the fourth quintile |
| 5 | In the fifth quintile |
| 6 | Greater than any value in the 30-year period |
| 7-14 | Reserved |
| 15 | Missing value |

0 15 025

Type of pollutant

| Code figure |  |
| --- | --- |
| 0 | Ozone |
| 1-14 | Reserved |
| 15 | Missing value |

0 19 001

- Type of synoptic feature

| Code figure |  |
| --- | --- |
| 0 | Depression or low (extratroplcal) |
| 1 | Tropical depression |
| 2 | Tropical storm |
| 3 | Severe tropical storm |
| 4 | Typhoon |
| 5-9 | Reserved |
| 10 | Dust/sandstorm |
| 11-62 | Reserved |
| 63 | Missing value |

Note:	New local names for storm of various strengths shall be added as necessary.

0 19 008

- Vertical extent of circulation

| Code figure |  |
| --- | --- |
| 0 | Reserved |
| 1 | Shallow (top of circulation below 700 hPa level) |
| 2 | Medium (top between 700 hPa and 400 hPa level) |
| 3 | Deep (top above 400 hPa level) |
| 4-6 | Reserved |
| 7 | Missing value |

0 19 010

Method for tracking the centre of synoptic feature

| Code figure |  |
| --- | --- |
| 1 | Minimum value of sea level pressure |
| 2 | Maximum value of 850 hPa relative vorticity |
| 3-14 | Reserved |
| 15 | Missing value |

0 20 003

- Present weather

| 00-49 | No precipitation at the station at the time of observation |
| --- | --- |
| 00-19 | No precipitation, fog, ice fog (except for 11 and 12), duststorm, sandstorm, drifting or blowing snow at the station* at the time of observation or, except for 09 and 17, during the preceding hour |

|  | Code figure |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| No Meteors except photometeors | 00 |  | Cloud development not observed or not observable |  |  | Characteristic change of the state of sky during the past hour |
|  | 01 |  | Clouds generally dissolving or becoming less developed. |  |  |  |
|  | 02 |  | State of sky on the whole unchanged. |  |  |  |
|  | 03 |  | Clouds generally forming or developing |  |  |  |
| Haze, dust, sand or smoke | 04 |  | Visibility reduced by smoke, e.g. veldt or forest fires, Industrial smoke or volcanic ashes |  |  |  |
|  | 05 |  | Haze |  |  |  |
|  | 06 |  | Widespread dust in suspension in the air, not raised by wind at or near the station at the time of observation |  |  |  |
|  | 07 |  | Dust or sand raised by wind at or near the station at the time of observation, but no well-developed dust whirl(s) or sand whirl(s), and no duststorm or sandstorm seen; or, In the case of sea stations and coastal stations, blowing spray at the station |  |  |  |
|  | 08 |  | Well-developed dust whirl(s) or sand whirl(s) seen at or near the station during the preceding hour or at the same time of observation, but no duststorm or sandstorm |  |  |  |
|  | 09 |  | Duststorm or sandstorm within sight at the time of observation, or at the station during the preceding hour |  |  |  |
|  | 10 |  | Mist |  |  |  |
|  | 11 |  | Patches |  | Shallow fog or Ice fog at the station, whether on land or sea, not deeper than about 2 metres on land or 10 metres at sea |  |
|  | 12 |  | More or less continuous |  |  |  |
|  | 13 |  | Lightning visible, no thunder heard |  |  |  |
|  | 14 |  | Precipitation within sight, not reaching the ground or the surface of the sea |  |  |  |
|  | 15 |  | Precipitation within sight, reaching the ground or the surface of the sea, but distant, i.e. estimated to be more than 5 km from the station |  |  |  |
|  | 16 |  | Precipitation within sight, reaching the ground or the surface of the sea, near to, but not at the station |  |  |  |
|  | 17 |  | Thunderstorm, but no precipitation at the time of observation |  |  |  |
|  | 18 |  | Squalls | At or within sight of the station during the preceding hour or at the time of observation |  |  |
|  | 19 |  | Funnel cloud(s)** |  |  |  |
| 20-29 |  | Precipitation, fog, Ice fog or thunderstorm at the station during the preceding hour but not at the time of observation |  |  |  |  |

| Code figure |  |  |  |  |
| --- | --- | --- | --- | --- |
| 20 | Drizzle (not freezing) or snow grains |  | not falling as shower(s) |  |
| 21 | Rain (not freezing) |  |  |  |
| 22 | Snow |  |  |  |
| 23 | Rain and snow or ice pellets |  |  |  |
| 24 | Freezing drizzle or freezing rain |  |  |  |
| 25 | Shower(s) of rain |  |  |  |
| 26 | Shower(s) of snow, or of rain and snow |  |  |  |
| 27 | Shower(s) of hail*, or of rain and hail* |  |  |  |
| 28 | Fog or Ice fog |  |  |  |
| 29 | Thunderstorm (with or without precipitation) |  |  |  |
| 30-39 | Duststorm, sandstorm, drifting or blowing snow |  |  |  |
| 30 | Slight or moderate duststorm or sandstorm |  | - has decreased during the preceding hour - no appreciable change during the preceding hour - has begun or has increased during the preceding hour |  |
| 31 |  |  |  |  |
| 32 |  |  |  |  |
| 33 | Severe duststorm or sandstorm |  | - has decreased during the preceding hour - no appreciable change during the preceding hour - has begun or has increased during the preceding hour |  |
| 34 |  |  |  |  |
| 35 |  |  |  |  |
| 36 | Slight or moderate drifting snow | generally low (below eye level) |  |  |
| 37 | Heavy drifting snow |  |  |  |
| 38 | Slight or moderate blowing snow | generally high(above eye level) |  |  |
| 39 | Heavy blowing snow |  |  |  |
| 40-49 | Fog or ice fog at the time of observation |  |  |  |
| 40 | Fog or ice fog at a distance at the time of observation, but not at the station during the preceding hour, the fog or ice fog extending to a level above that of the observer |  |  |  |
| 41 | Fog or ice fog in patches |  |  |  |
| 42 | Fog or ice fog, sky visible |  | has become thinner during the preceding hour |  |
| 43 | Fog or ice fog, sky invisible |  |  |  |
| 44 | Fog or ice fog, sky visible |  | no appreciable change during the preceding hour |  |
| 45 | Fog or ice fog, sky invisible |  |  |  |
| 46 | Fog or ice fog, sky visible |  | has begun or has become thicker during the preceding hour |  |
| 47 | Fog or ice fog, sky invisible |  |  |  |
| 48 | Fog, depositing rime, sky visible |  |  |  |
| 49 | Fog, depositing rime, sky invisible |  |  |  |
| 50-99 | Precipitation at the station at the time of observation |  |  |  |
| 50-59 | Drizzle |  |  |  |
| 50 | Drizzle, not freezing, intermittent |  |  | slight at time of observation |
| 51 | Drizzle, not freezing, continuous |  |  |  |
| 52 | Drizzle, not freezing, intermittent |  |  | moderate at time of observation |
| 53 | Drizzle, not freezing, continuous |  |  |  |
| 54 | Drizzle, not freezing, intermittent |  |  | heavy (dense) at time of observation |
| 55 | Drizzle, not freezing, continuous |  |  |  |
| 56 | Drizzle, freezing, slight |  |  |  |
| 57 | Drizzle, freezing, moderate or heavy (dense) |  |  |  |
| 58 | Drizzle and rain, slight |  |  |  |
| 59 | Drizzle and rain, moderate or heavy |  |  |  |

| Code figure |  |  |  |
| --- | --- | --- | --- |
| 60-69 | Rain |  |  |
| 60 | Rain, not freezing, intermittent | slight at time of observation |  |
| 61 | Rain, not freezing, continuous |  |  |
| 62 | Rain, not freezing, intermittent | moderate at time observation |  |
| 63 | Rain, not freezing, continuous |  |  |
| 64 | Rain, not freezing, intermittent | heavy at time of observation |  |
| 65 | Rain, not freezing, continuous |  |  |
| 66 | Rain, freezing, slight |  |  |
| 67 | Rain, freezing, moderate or heavy |  |  |
| 68 | Rain or drizzle and snow, light |  |  |
| 69 | Rain or drizzle and snow, moderate or heavy |  |  |
| 70-79 | Solid precipitation not in showers |  |  |
| 70 | Intermittent fall of snowflakes | slight at time of observation |  |
| 71 | Continuous fall of snowflakes |  |  |
| 72 | Intermittent fall of snowflakes | moderate at time of observation |  |
| 73 | Continuous fall of snowflakes |  |  |
| 74 | Intermittent fall of snowflakes | heavy at time of observation |  |
| 75 | Continuous fall of snowflakes |  |  |
| 76 | Diamond dust (with or without fog) |  |  |
| 77 | Snow grains (with or without fog) |  |  |
| 78 | Isolated star-like snow crystals (with or without fog) |  |  |
| 79 | Ice pellets |  |  |
| 80-99 | Showery precipitation, or precipitation with current or recent thunderstorm |  |  |
| 80 | Rain shower(s), slight |  |  |
| 81 | Rain shower(s), moderate or heavy |  |  |
| 82 | Rain shower(s), violent |  |  |
| 83 | Shower(s) of rain and snow mixed, slight |  |  |
| 84 | Shower(s) of rain and snow mixed, moderate or heavy |  |  |
| 85 | Snow shower(s), slight |  |  |
| 86 | Snow shower(s), moderate or heavy |  |  |
| 87 | Shower(s) of snow pellets or small hail, with or without rain or rain and snow mixed |  | - slight |
| 88 |  |  | - moderate or heavy |
| 89 | Shower(s) of hail, with or without rain or rain and snow mixed, not associated with thunder |  | - slight |
| 90 |  |  | - moderate or heavy |
| 91 | Slight rain at time of observation |  | Thunderstorm during the preceding hour but not at time of observation |
| 92 | Moderate or heavy rain at time of observation. |  |  |
| 93 | Slight snow, or rain and snow mixed or hail* at time of observation. |  |  |
| 94 | Moderate or heavy snow, or rain and snow mixed or hail* at time of observation. |  |  |
| 95 | Thunderstorm, slight or moderate, without hail*, but with rain and/or snow at time of observation |  | Thunderstorm at time of observation. |
| 96 | Thunderstorm, slight or moderate, with hail* at time of observation. |  |  |
| 97 | Thunderstorm, heavy, without hail*, but with rain and/or snow at time of observation. |  |  |
| 98 | Thunderstorm combined with duststorm or sandstorm at time of observation. |  |  |
| 99 | Thunderstorm, heavy, with hail* at time of observation. |  |  |

| Present weather reported from an automatic weather station |  |
| --- | --- |
| Code figure |  |
| 100 | No significant weather observed |
| 101 | Clouds generally dissolving or becoming less developed during the past hour |
| 102 | State of sky on the whole unchanged during the past hour |
| 103 | Clouds generally forming or developing during the past hour |
| 104 | Haze or smoke, or dust in suspension in the air, visibility equal to, or greater than, 1km |
| 105 | Haze or smoke, or dust In suspension in the air, visibility less than 1 km |
| 106-109 | Reserved |
| 110 | Mist |
| 111 | Diamond dust |
| 112 | Distant lightning |
| 113-117 | Reserved |
| 118 | Squalls |
| 119 | Reserved |
|  | Code figures 120-126 are used to report precipitation, fog (or ice fog) or thunderstorm at the station during the preceding hour but not at the time of observation |
| 120 | Fog |
| 121 | PRECIPITATION |
| 122 | Drizzle (not freezing) or snow grains |
| 123 | Rain (not freezing) |
| 124 | Snow |
| 125 | Freezing drizzle or freezing rain |
| 126 | Thunderstorm (with or without precipitation) |
| 127 | Blowing OR DRIFTING SNOW OR SAND |
| 128 | Blowing or drifting snow or sand, visibility equal to, or greater than, 1 km |
| 129 | Blowing or drifting snow or sand, visibility less than 1 km |
| 130 | FOG |
| 131 | Fog or ice fog In patches |
| 132 | Fog or ice fog, has become thinner during the past hour |
| 133 | Fog or ice fog, no appreciable change during the past hour |
| 134 | Fog or ice fog, has begun or become thicker during the past hour |
| 135 | Fog, depositing rime |
| 136-139 | Reserved |
| 140 | PRECIPITATION |
| 141 | Precipitation, slight or moderate |
| 142 | Precipitation, heavy |
| 143 | Liquid precipitation, slight or moderate |
| 144 | Liquid precipitation, heavy |
| 145 | Solid precipitation, slight or moderate |
| 146 | Solid precipitation, heavy |
| 147 | Freezing precipitation, slight or moderate |
| 148 | Freezing precipitation, heavy |
| 149 | Reserved |
| 150 | DRIZZLE |
| 151 | Drizzle, not freezing, slight |
| 152 | Drizzle, not freezing, moderate |
| 153 | Drizzle, not freezing, heavy |
| 154 | Drizzle, freezing, slight |
| 155 | Drizzle, freezing, moderate |
| 156 | Drizzle, freezing, heavy |
| 157 | Drizzle and rain, slight |
| 158 | Drizzle and rain, moderate or heavy |

| Code figure |  |
| --- | --- |
| 159 | Reserved |
| 160 | RAIN |
| 161 | Rain, not freezing, slight |
| 162 | Rain, not freezing, moderate |
| 163 | Rain, not freezing, heavy |
| 164 | Rain, freezing, slight |
| 165 | Rain, freezing, moderate |
| 166 | Rain, freezing, heavy |
| 167 | Rain (or drizzle) and snow, slight |
| 168 | Rain (or drizzle) and snow, moderate or heavy |
| 169 | Reserved |
| 170 | SNOW |
| 171 | Snow, slight |
| 172 | Snow, moderate |
| 173 | Snow, heavy |
| 174 | Ice pellets, slight |
| 175 | Ice pellets, moderate |
| 176 | Ice pellets, heavy |
| 177-179 | Reserved |
| 180 | SHOWER(S) or intermittent PRECIPITATION |
| 181 | Rain shower(s) or intermittent rain, slight |
| 182 | Rain shower(s) or intermittent rain, moderate |
| 183 | Rain shower(s) or intermittent rain, heavy |
| 184 | Rain shower(s) or intermittent rain, violent |
| 185 | Snow shower(s) or intermittent snow, slight |
| 186 | Snow shower(s) or intermittent snow, moderate |
| 187 | Snow shower(s) or intermittent snow, heavy |
| 188-189 | Reserved |
| 190 | THUNDERSTORM |
| 191 | Thunderstorm, slight or moderate, with no precipitation |
| 192 | Thunderstorm, slight or moderate, with rain showers and/or snow showers |
| 193 | Thunderstorm, slight or moderate, with hail |
| 194 | Thunderstorm, heavy, with no precipitation |
| 195 | Thunderstorm, heavy, With rain showers and/or snow showers |
| 196 | Thunderstorm, heavy, with hail |
| 197-198 | Reserved |
| 199 | Tornado |
|  | Present weather (In addition to present weather report from either a manned or an automatic station) |
| Decile 200-209 |  |
| 200 | Not used |
| 201 | Not used |
| 202 | Not used |
| 203 | Not used |
| 204 | Volcanic ash suspended In the air aloft |
| 205 | Not used |
| 206 | Thick dust haze, visibility less than 1 km |
| 207 | Blowing spray at the station |
| 208 | Drifting dust (sand) |
| 209 | Wall of dust or sand in distance (like haboob) |

| Code figure |  |  |
| --- | --- | --- |
| Decile 210-219 |  |  |
| 210 | Snow haze |  |
| 211 | Whiteout |  |
| 212 | Not used |  |
| 213 | Lightning, cloud to surface |  |
| 214-216 | Not used |  |
| 217 | Dry thunderstorm |  |
| 218 | Not used |  |
| 219 | Tornado cloud (destructive) at or within sight of the station during preceding hour or at the time of observation |  |
| Decile 220-229 |  |  |
| 220 | Deposition of volcanic ash |  |
| 221 | Deposition of dust or sand |  |
| 222 | Deposition of dew |  |
| 223 | Deposition of wet snow |  |
| 224 | Deposition of soft rime |  |
| 225 | Deposition of hard rime |  |
| 226 | Deposition of hoarfrost |  |
| 227 | Deposition of glaze |  |
| 228 | Deposition of ice crust (ice slick) |  |
| 229 | Not used |  |
| Decile 230-239 |  |  |
| 230 | Duststorm or sandstorm with temperature below 00C |  |
| 231-238 | Not used |  |
| 239 | Blowing snow, impossible to determine whether snow is falling or not |  |
| Decile 240-249 |  |  |
| 240 | Not used |  |
| 241 | Fog on sea |  |
| 242 | Fog in valleys |  |
| 243 | Arctic or Antarctic sea smoke |  |
| 244 | Steam fog (sea, lake or river) |  |
| 245 | Steam log (land) |  |
| 246 | Fog over ice or snow cover |  |
| 247 | Dense fog, visibility 60-90 m |  |
| 248 | Dense fog, visibility 30-60 m |  |
| 249 | Dense fog, visibility less than 30 m |  |
| Decile 250-259 |  |  |
| 250 | Drizzle, rate of fall | less than 0.10 mm h-1 |
| 251 |  | 0.10-0.19 mm h-1 |
| 252 |  | 0.20-0.39 mm h-1 |
| 253 |  | O.40-0.79 mm h-1 |
| 254 |  | O.80-1.59 mm h-1 |
| 255 |  | l.60-3.19 mm h-1 |
| 256 |  | 3.20-6.39 mm h-1 |
| 257 |  | 6.4 mm h-1 or more |
| 258 | Not used |  |
| 259 | Drizzle and snow |  |

| Code figure |  |  |  |
| --- | --- | --- | --- |
| Decile 260-269 |  |  |  |
| 260 | Rain, rate of fall | less than 1.0 mm h-1 |  |
| 261 |  | 1.- 1.9 mm h-1 |  |
| 262 |  | 2.- 3.9 mm h-1 |  |
| 263 |  | 4.- 7.9 mm h-1 |  |
| 264 |  | 8.-15.9 mm h-1 |  |
| 265 |  | 16.0-31.9 mm h-1 |  |
| 266 |  | 32.0-63.9 mm h-1 |  |
| 267 |  | 64.0 mm h-1 or more |  |
| 268-269 | Not used |  |  |
| Decile 270-279 |  |  |  |
| 270 | Snow, rate of fall | less than 1.0 cm h-1 |  |
| 271 |  | 1.0-1.9 cm h-1 |  |
| 272 |  | 2.0-3.9 cm h-1 |  |
| 273 |  | 4.0-7.9 cm h-1 |  |
| 274 |  | 8.0-15.9 cm h-1 |  |
| 275 |  | 16.0-31.9 cm h-1 |  |
| 276 |  | 32.0-63.9 cm h-1 |  |
| 277 |  | 64.0 cm h-1 or more |  |
| 278 | Snow or Ice crystal precipitation from a clear sky |  |  |
| 279 | Wet snow, freezing on contact |  |  |
| Decile 280-299 |  |  |  |
| 280 | Precipitation of rain |  |  |
| 281 | Precipitation of rain, freezing |  |  |
| 282 | Precipitation of rain and snow mixed. |  |  |
| 283 | Precipitation of snow |  |  |
| 284 | Precipitation of snow pellets or small hall |  |  |
| 285 | Precipitation of snow pellets or small hail, with rain |  |  |
| 286 | Precipitation of snow pellets or small hail, with rain and snow mixed |  |  |
| 287 | Precipitation of snow pellets or small hail, with snow |  |  |
| 288 | Precipitation of hail |  |  |
| 289 | Precipitation of hail, with rain |  |  |
| 290 | Precipitation of hall, with rain and snow mixed |  |  |
| 291 | Precipitation of hail, with snow |  |  |
| 292 | Shower(s) or thunderstorm over sea |  |  |
| 293 | Shower(s) or thunderstorm over mountains |  |  |
| 294-299 | Not used |  |  |
| 300-507 | Reserved |  |  |
| 508 | No significant phenomenon to report, present and past weather omitted |  |  |
| 509 | No observation, data not available, present and past weather omitted |  |  |
| 510 | Present and past weather missing, but expected |  |  |
| 511 | Missing value |  |  |

Notes:

- The middle portion of this code table (code figures 100-199) includes terms on several levels to cover simple and increasingly complex automatic stations.

- Generic terms for weather (e.g. fog, drizzle) are intended for use at automatic stations capable of determining types of weather but no other information.Generic terms are included in the code table using all capital letters.

(3)	Code figures for generic precipitation (code figures 140-148) are arranged in order of increasing complexity. For example, a very simple station that can sense only the presence or absence of precipitation would use code figure 140 (precipitation). At the next level, an automatic station capable of sensing amount but not type would use code figure 141 or 142. An automatic station capable of sensing gross type (liquid, solid, freezing) and amount would use code figures 143-148. An automatic station capable of reporting actual types of precipitation (e.g. drizzle rain), but not the amount, would use the appropriate whole decile number (e.g. 150 for generic drizzle, 160 for generic rain).

0 20 004/0 20 005

- Past weather (l) and (2)

| Code figure |  |
| --- | --- |
| 0 | Cloud covering 1/2 or less of the sky throughout the appropriate period |
| 1 | Cloud coverIng more than 1/2 of the sky during part of the appropriate period and covering 1/2 or less during part of the period |
| 2 | Cloud covering more than 1/2 of the sky throughout the appropriate period |
| 3 | Sandstorm, duststorm or blowing snow |
| 4 | Fog or ice fog or thick haze |
| 5 | Drizzle |
| 6 | Rain |
| 7 | Snow, or rain and snow mixed |
| 8 | Shower(s) |
| 9 | Thunderstorm(s) with or without precipitation |
| 10 | No significant weather observed |
| 11 | Visibility REDUCED |
| 12 | Blowing phenomena, visibility reduced |
| 13 | FOG |
| 14 | PRECIPITATION |
| 15 | Drizzle |
| 16 | Rain |
| 17 | Snow or ice pellets |
| 18 | Showers or intermittent precipitation |
| 19 | Thunderstorm |
| 20-30 | Reserved |
| 31 | Missing value |

Note: 	The weather descriptions in code figures 10 to 19 are progressively complex, to accommodate the different levels of weather discrimination capability of various automatic stations. Stations having only basic sensing capability may use the lower code figures and basic generic descriptions (shown in capital letters).  Stations with progressively higher discrimination capability shall use the more detailed descriptions (higher codes).

0 20 008

- Cloud Distribution for Aviation

| code figure |  |  |
| --- | --- | --- |
| 0 | Sky Clear |  |
| 1 | Few |  |
| 2 | Scattered |  |
| 3 | Broken |  |
| 4 | Overcast |  |
| 5 | Reserved |  |
| 6 | Scattered/Broken | (Many forecasts use Scattered/Broken or |
| 7 | Broken/Overcast | (Broken/Overcast followed by cloud type(s)) |
| 8 | Isolated | (Used on aviation charts to describe the cloud type Cb) |
| 9 | Isolated embedded | (Used on aviation charts to describe the cloud type Cb) |
| 10 | Occasional | (Used on aviation charts to describe the cloud type Cb) |
| 11 | Occasional embedded | (Used on aviation charts to describe the cloud type Cb) |
| 12 | Frequent | (Used on aviation charts to describe the cloud type Cb) |
| 13 | Dense | (Used on aviation charts to describe cloud that would cause sudden changes in visibility (less than 1000m)) |
| 14 | Layers |  |
| 15 | Obscured (OBSC) (VAL) |  |
| 16 | Embedded (EMBD) (VAL) |  |
| 17 | Frequent embedded (VAL) |  |
| 18-30 | Reserved |  |
| 31 | Missing value |  |

VAL = code figure in validation process

0 20 009

- General Weather Indicator (TAF/METAR)

| Code figure |  |
| --- | --- |
| 0 | Reserved |
| 1 | NSC Nil Significant Cloud |
| 2 | CAVOK |
| 3 | SKC Sky Clear |
| 4 | NSW Nil Significant Weather |
| 5-14 | Reserved |
| 15 | Missing value |

0 20 011

- Cloud amount

| Code figure |  |  |
| --- | --- | --- |
| 0 | 0 | 0 |
| 1 | 1 okta or less, but not zero | 1/10 or less, but not zero |
| 2 | 2 oktas | 2/10 - 3/10 |
| 3 | 3 oktas | 4/10 |
| 4 | 4 oktas | 5/10 |
| 5 | 5 oktas | 6/10 |
| 6 | 6 oktas | 7/10 - 8/10 |
| 7 | 7 oktas or more, but not 8 oktas | 9/10 or more, but not 10/10 |
| 8 | 8 oktas | 10/10 |
| 9 | Sky obscured by fog and/or other meteorological phenomena |  |
| 10 | Sky partially obscured by fog and/or other meteorological phenomena |  |
| 11 | Scattered |  |
| 12 | Broken |  |
| 13 | Few |  |
| 14 | Reserved |  |
| 15 | Cloud cover is Indiscernible for reasons other than fog or other meteorological phenomena, or observation is not made |  |

Notes:

(1)	For use of code figure 15, see Regulation 12.1.4.

(2)	`Clear' and `overcast' are coded by 0 and 8 respectively.

0 20 012

- Cloud type

| Code figure |  |
| --- | --- |
| 0 | Cirrus (Ci) |
| 1 | Cirrocumulus (Cc) |
| 2 | Cirrostratus (Cs) |
| 3 | Altocumulus (Ac) |
| 4 | Altostratus (As) |
| 5 | Nimbostratus (Ns) |
| 6 | Stratocumulus (Sc) |
| 7 | Stratus(St) |
| 8 | Cumulus (Cu) |
| 9 | Cumulonimbus (Cb) |
| 10 | No CH clouds |
| 11 | Cirrus fibratus, sometimes uncinus, not progressively invading the sky |
| 12 | Cirrus spissatus, in patches or entangled sheaves, which usually do not increase and sometimes seem to be the remains of the upper part of a Cumulonimbus; or Cirrus castellanus or floccus |
| 13 | Cirrus spissatus cumulonimbogenitus |
| 14 | Cirrus uncinus or fibratus, or both, progressively invading the sky; they generally thicken as a whole |
| 15 | Cirrus (often in bands) and Cirrostratus, or Cirrostratus alone, progressively invading the sky; they generally thicken as a whole, but the continuous veil does not reach 45 degrees above the horizon |
| 16 | Cirrus (often In bands) and Cirrostratus, or Cirrostratus alone, progressively Invading the sky; they generally thicken as a whole; the continuous veil extends more than 45 degrees above the horizon, without the sky being totally covered |
| 17 | Cirrostratus covering the whole sky |
| 18 | Cirrostratus not progressively invading the sky and not entirely covering It |
| 19 | Cirrocumulus alone, or Cirrocumulus predominant among the CH clouds |
| 20 | No CM clouds |
| 21 | Altostratus translucidus |
| 22 | Altostratus opacus or Nimbostratus |
| 23 | Altocumulus translucidus at a single level |
| 24 | Patches (often Lenticular) of Altocumulus translucidus, continually changing and occurring at one or more levels |
| 25 | Altocumulus translucidus in bands, or one or more layers of Altocumulus translucidus or opacus, progressively Invading the sky;these Altocumulus clouds generally thicken as a whole |
| 26 | Altocumulus cumulogenitus (or cumulonimbogenitus) |
| 27 | Altocumulus translucidus or opacus In two or more layers, or Altocumulus opacus In a single layer, not progressively Invading the sky, or Altocumulus with Altostratus or Nimbostratus |
| 28 | Altocumulus castellanus or floccus |
| 29 | Altocumulus of a chaotic sky, generally at several levels |
| 30 | No CL clouds |
| 31 | Cumulus humilis or Cumulus fractus other than of bad weather,* or both |
| 32 | Cumulus mediocris or congestus, Towering cumulus (TCU), with or without Cumulus of species fractus or humilis or Stratocumulus, all having their bases at the same level |
| 33 | Cumulonimbus calvus, with or without Cumulus, Stratocumulus or Stratus |
| 34 | Stratocumulus cumulogenitus |
| 35 | Stratocumulus other than Stratocumulus cumulogenitus |
| 36 | Stratus nebulosus or Stratus fractus other than of bad weather*, or both |
| 37 | Stratus fractus or Cumulus fractus of bad weather*, or both (pannus),usually below Altostratus or Nimbostratus |
| 38 | Cumulus and Stratocumulus other than Stratocumulus cumulogenitus, with bases at different levels |
| 39 | Cumulonimbus capillatus (often with an anvil), with or without Cumulonimbus calvus, Cumulus, Stratocumulus, Stratus or pannus |
| 40 | CH |
| 41 | CM |
| 42 | CL |
| 43-58 | Reserved |
| 59 | Cloud not visible owing to darkness, fog, duststorm, sandstorm, or other analogous phenomena |
| 60 | CH clouds Invisible owing to darkness, fog, blowing dust or sand, or other similar phenomena, or because of a continuous layer of lower clouds |
| 61 | CM clouds Invisible owing to darkness, fog, blowing dust or sand, or other similar phenomena, or because of continuous layer of lower clouds |
| 62 | CL clouds invisible owing to darkness, fog, blowing dust or sand, or other similar phenomena |
| 63 | Missing value |

*	"Bad weather" denotes the conditions which generally exist during precipitation and a short time before and after.

0 20 017

- Cloud top description

| Code figure |  |  |
| --- | --- | --- |
| 0 | Isolated cloud fragments of clouds |  |
| 1 | Continuous cloud | flat tops |
| 2 | Broken cloud - small breaks |  |
| 3 | Broken cloud - large breaks |  |
| 4 | Continuous cloud | undulating tops |
| 5 | Broken cloud - small breaks |  |
| 6 | Broken cloud - large breaks |  |
| 7 | Continuous or almost continuous waves with towering clouds above the top of the layer |  |
| 8 | Groups of waves with towering clouds above the top of the layer |  |
| 9 | Two or more layers at different levels |  |
| 10-14 | reserved |  |
| 15 | Missing value |  |

0 20 018

- Tendency of runway visual range

| Code figure |  |
| --- | --- |
| 0 | Increasing (U) |
| 1 | Decreasing (D) |
| 2 | No distinct change (N) |
| 3 | Missing value |

0 20 021

Type of precipitation

| Bit No. |  |
| --- | --- |
| 1 | Precipitation-unknown type |
| 2 | Liquid precipitation not freezing |
| 3 | Liquid freezing precipitation |
| 4 | Drizzle |
| 5 | Rain |
| 6 | Solid precipitation |
| 7 | Snow |
| 8 | Snow grains |
| 9 | Snow pellets |
| 10 | Ice pellets |
| 11 | Ice crystals |
| 12 | Diamond dust |
| 13 | Small hail |
| 14 | Hail |
| 15 | Glaze |
| 16 | Rime |
| 17 | Soft rime |
| 18 | Hard rime |
| 19 | Clear ice |
| 20 | Wet snow |
| 21 | Hoar frost |
| 22 | Dew |
| 23 | White dew |
| 24-29 | Reserved |
| All 30 | Missing value |

Note:	Mixed precipitation is indicated by setting to one the bits of all the observed single types of precipitation

0 20 022

Character of precipitation

| Code figure |  |
| --- | --- |
| 0 | No precipitation |
| 1 | Continuous |
| 2 | Intermittent |
| 3 | Shower |
| 4 | Not reaching ground |
| 5 | Deposition |
| 6-14 | Reserved |
| 15 | Missing value |

0 20 023

Other weather phenomena

| Bit No. |  |
| --- | --- |
| 1 | Dust/sand whirl |
| 2 | Squalls |
| 3 | Sand storm |
| 4 | Dust storm |
| 5 | Lightning - cloud to surface |
| 6 | Lightning - cloud to cloud |
| 7 | Lightning - distant |
| 8 | Thunderstorm |
| 9 | Funnel Cloud not touching surface |
| 10 | Funnel cloud touching surface |
| 11 | Spray |
| 12-17 | Reserved |
| All 18 | Missing value |

0 20 024

Intensity of phenomena

| Code figure |  |
| --- | --- |
| 0 | No phenomena |
| 1 | Light |
| 2 | Moderate |
| 3 | Heavy |
| 4 | Violent |
| 5 | Severe (VAL) |
| 6 | Reserved |
| 7 | Missing value |

VAL = code figure in validation process

0 20 025

Obscuration

| Bit No. |  |
| --- | --- |
| 1 | Fog |
| 2 | Ice fog |
| 3 | Steam fog |
| 4-6 | Reserved |
| 7 | Mist |
| 8 | Haze |
| 9 | Smoke |
| 10 | Volcanic ash |
| 11 | Dust |
| 12 | Sand |
| 13 | Snow |
| 14-20 | Reserved |
| All 21 | Missing value |

0 20 026

Character of obscuration

| Code figure |  |
| --- | --- |
| 0 | No change |
| 1 | Shallow |
| 2 | Patches |
| 3 | Partial |
| 4 | Freezing |
| 5 | Low drifting |
| 6 | Blowing |
| 7 | Increasing |
| 8 | Decreasing |
| 9 | In suspension in the air |
| 10 | Wall |
| 11 | Dense |
| 12 | Whiteout |
| 13-14 | Reserved |
| 15 | Missing value |

0 20 027

Phenomena occurrence

| Bit No. |  |
| --- | --- |
| 1 | At time of observation |
| 2 | In past hour |
| 3 | In time period for past weather W1W2 |
| 4 | In time period specified |
| 5-6 | Reserved |
| 7 | At the station |
| 8 | In the vicinity |
| All 9 | Missing value |

Note :	Phenomenon in 0 20 027 means any phenomenon, including precipitation and obscuration.

0 20 029

Rain flag

| Code figure |  |
| --- | --- |
| 0 | No rain |
| 1 | Rain |
| 2 | Reserved |
| 3 | Missing value |

0 20 032

- Rate of ice accretion

| Code figure |  |
| --- | --- |
| 0 | Ice not building up |
| 1 | Ice building up slowly |
| 2 | Ice building up rapidly |
| 3 | Ice melting or breaking up slowly |
| 4 | Ice melting or breaking up rapidly |
| 5-6 | Reserved |
| 7 | Missing value |

0 20 033

- Cause of ice accretion

| Bit No. |  |
| --- | --- |
| 1 | Icing from ocean spray |
| 2 | Icing from fog |
| 3 | Icing from rain |
| All 4 | Missing value |

0 20 034

- Sea Ice concentration

| Code figure |  |  |  |
| --- | --- | --- | --- |
| 0 | No sea ice In sight |  |  |
| 1 | Ship in open lead more than 1.0 nautical mile wide, or ship in fast ice with boundary beyond limit of visibility |  |  |
| 2 | Sea ice present In concentrations less than 3/10 (3/8) open water or very open pack ice | Sea ice concentration is uniform in the observation area | Ship in ice or within 0.5 nautical mile of ice edge |
| 3 | 4/10 to 6/10 (3/8 to less than 6/8), open pack ice |  |  |
| 4 | 7/10 to 8/10 (6/8 to less 7/8), close pack ice |  |  |
| 5 | 9/10 or more, but not 10/10 (7/8 to less than 8/8), very close pack ice |  |  |
| 6 | Strips and patches of pack ice with open water between | Sea ice concentration is not uniform in the observation area |  |
| 7 | Strips and patches of close or very close pack ice with areas of lesser concentration between |  |  |
| 8 | Fast ice with open water, very open or open pack Ice to seaward of the ice boundary |  |  |
| 9 | Fast ice with close or very close pack ice to seaward of the boundary |  |  |
| 10-13 | Reserved |  |  |
| 14 | Unable to report, because of darkness, lack of visibility, or because ship is more than 0.5 nautical mile away from ice edge |  |  |
| 15-30 | Reserved |  |  |
| 31 | Missing value |  |  |

0 20 035

- Amount and type of Ice

| Code figure |  |
| --- | --- |
| 0 | No ice of land origin |
| 1 | 1-5 icebergs, no growlers or bergy bits |
| 2 | 6-10 icebergs, no growlers or bergy bits |
| 3 | 11-20 Icebergs, no growlers or bergy bits |
| 4 | Up to and Including 10 growlers and bergy bits - no icebergs |
| 5 | More than 10 growlers and bergy bits - no Icebergs |
| 6 | 1-5 icebergs, with growlers and bergy bits |
| 7 | 6-10 icebergs, with growlers and bergy bits |
| 8 | 11-20 icebergs, with growlers and bergy bits |
| 9 | More than 20 Icebergs, with growlers and bergy bits - a major hazard to navigation |
| 10-13 | Reserved |
| 14 | Unable to report, because of darkness, lack of visibility or because only sea ice is visible |
| 15 | Missing value |

0 20 036

- Ice situation

| Code figure |  |
| --- | --- |
| 0 | Ship in open water with floating ice in sight |
| 1 | Ship In easily penetrable ice; conditions improving |
| 2 | Ship In easily penetrable ice; conditions not changing |
| 3 | Ship in easily penetrable ice; conditions worsening |
| 4 | Ship in ice difficult to penetrate; conditions improving |
| 5 | Ship in ice difficult to penetrate; conditions not changing |
| 6 | Ship in ice difficult to penetrate and conditions worsening. Ice forming and floe freezing together |
| 7 | Ship in ice difficult to penetrate and conditions worsening. Ice under slight pressure |
| 8 | Ship in ice difficult to penetrate and conditions worsening. Ice under moderate or severe pressure |
| 9 | Ship in ice difficult to penetrate and conditions worsening. Ship beset |
| 10-29 | Reserved |
| 30 | Unable to report, because of darkness or lack of visibility |
| 31 | Missing value |

0 20 037

- Ice development

| Code figure |  |
| --- | --- |
| 0 | New ice only (frazil ice, grease ice, slush, shuga) |
| 1 | Nilas or ice rind, less than 10 cm thick |
| 2 | Young ice (grey ice, grey-white ice), 10-30 cm thick |
| 3 | Predominantly new and/or young ice with some first-year ice |
| 4 | Predominantly thin first-year ice with some new and/or young ice |
| 5 | All thin first-year ice (30-70 cm thick) |
| 6 | Predominantly medium first-year ice (70-120 cm thick) and thick first-year ice (>120 cm thick) with some thinner (younger) first-year ice |
| 7 | All medium and thick first-year ice |
| 8 | Predominantly medium and thick first-year ice with some old ice (usually more than 2 metres thick) |
| 9 | Predominantly old ice |
| 11-29 | Reserved |
| 30 | Unable to report, because of darkness, lack of visibility or because only ice of land origin is visible or because ship is more than 0.5 nautical mile away from ice edge |
| 31 | Missing value |

0 20 041

- Airframe Icing

| Code figure |  |
| --- | --- |
| 0 | No icing |
| 1 | Light icing |
| 2 | Light icing In cloud |
| 3 | Light icing In precipitation |
| 4 | Moderate icing |
| 5 | Moderate icing in cloud |
| 6 | Moderate icing in precipitation |
| 7 | Severe icing |
| 8 | Severe icing in cloud |
| 9 | Severe icing in precipitation |
| 10 | Trace of icing |
| 11 | Trace of icing in cloud |
| 12 | Trace of icing in precipitation |
| 13-14 | Reserved |
| 15 | Missing value |

0 20 042

- Airframe Icing present

| Code figure |  |
| --- | --- |
| 0 | No icing |
| 1 | Icing present |
| 2 | Reserved |
| 3 | Missing value |

0 20 045

- Supercooled large droplet (SLD) conditions

| Code figure |  |
| --- | --- |
| 0 | No SLD conditions present |
| 1 | SLD conditions present |
| 2 | Reserved |
| 3 | Missing value |

0 20 050

- Cloud index

| Code figure |  |
| --- | --- |
| 0 | Reserved |
| 1 | 1st low cloud |
| 2 | 2nd low cloud |
| 3 | 3rd low cloud |
| 4 | 1st medium cloud |
| 5 | 2nd medium cloud |
| 6 | 3rd medium cloud |
| 7 | 1st high cloud |
| 8 | 2nd high cloud |
| 9-254 | Reserved |
| 255 | Missing value |

0 20 056

- Cloud phase

| Code figure |  |
| --- | --- |
| 0 | Unknown |
| 1 | Water |
| 2 | Ice |
| 3 | Mixed |
| 4-6 | Reserved |
| 7 | Missing value |

0 20 062

- State of the ground

| Code figure |  |  |
| --- | --- | --- |
| 0 | Surface of ground dry (without cracks and no appreciable amount of dust or loose sand) | without snow or measurable ice cover |
| 1 | Surface of ground moist |  |
| 2 | Surface of ground wet (standing water in small or large pools on surface) |  |
| 3 | Flooded |  |
| 4 | Surface of ground frozen |  |
| 5 | Glaze on ground |  |
| 6 | Loose dry dust or sand not covering ground completely |  |
| 7 | Thin cover of loose dry dust or sand covering ground completely |  |
| 8 | Moderate or thick cover of loose dry dust or sand covering ground completely |  |
| 9 | Extremely dry with cracks |  |
| 10 | Ground predominantly covered by ice |  |
| 11 | Compact or wet snow (with or without ice) covering less than one-half of the ground | with snow or measurable ice cover |
| 12 | Compact or wet snow (with or without ice) covering at least one-half of the ground but ground not completely covered |  |
| 13 | Even layer of compact or wet snow covering ground completely |  |
| 14 | Uneven layer of compact or wet snow covering ground completely |  |
| 15 | Loose dry snow covering less than one-half of the ground |  |
| 16 | Loose dry snow covering at least one-half of the ground (but not completely) |  |
| 17 | Even layer of loose dry snow covering ground completely |  |
| 18 | Uneven layer of loose dry snow covering ground completely |  |
| 19 | Snow covering ground completely; deep drifts |  |
| 20-30 | Reserved |  |
| 31 | Missing value |  |

Notes:

(1)	The definitions in code numbers 0 to 2 and 4 apply to representative bare ground and numbers 3, 5 to 9 and 10 to 19 to an open representative area.

(2)	In all instances the highest code figures applicable are to be reported.

(3)	In the above code table, whenever reference is made to ice, it also includes solid precipitation other than snow.

0 20 063

- Special phenomena

(To be developed)

0 20 071

Accuracy of fix and rate of atmospherics

| Code figure | Accuracy of fix (estimated error) | Repetition rate |
| --- | --- | --- |
| 0 | No assessment | No assessment |
| 1 | Less than 50 km | Less than 1 per second |
| 2 | Between 50 and 200 km | Less than 1 per second |
| 3 | More than 200 km | Less than 1 per second |
| 4 | Less than 50 km | 1 or more per second |
| 5 | Between 50 and 200 km | 1 or more per second |
| 6 | More than 200 km | 1 or more per second |
| 7 | Less thank 50 km | Rate so rapid number cannot be counted |
| 8 | Between 50 and 200 km | Rate so rapid number cannot be counted |
| 9 | More than 200 km | Rate so rapid number cannot be counted |
| 10-14 | Reserved |  |
| 15 | Missing value |  |

0 20 090

- Special clouds

| Code figure |  |
| --- | --- |
| 0 | Reserved |
| 1 | Nacreous clouds |
| 2 | Noctilucent clouds |
| 3 | Clouds from waterfalls |
| 4 | Clouds from fires |
| 5 | Clouds from volcanic eruptions |
| 6-14 | Reserved |
| 15 | Missing value |

0 21 066

- Wave Scatterometer Product Confidence

| Bit No. |  |
| --- | --- |
| 1 | Process equipment not working |
| 2 | Equipment failed |
| 3 | PRF code changed during image generation |
| 4 | Sampling window changed during image generation |
| 5 | Gain changed during image generation |
| 6 | Chirp replica exceeds specified values |
| 7 | Input data mean & standard deviation of in-phase & quadrature out of range |
| 8 | Doppler centroid confidence > MMCC value |
| 9 | Doppler centroid absolute value > PRF/2 |
| 10 | Doppler ambiguity confidence < MMCC value |
| 11 | Output data mean and standard deviation =< MMCC value |
| All 12 | Missing value |

Notes:

(1)	MMCC is Mission Management Control Centre

(2)	PRF is Pulse Repitition Frequency

0 21 067

- Wind Scatterometer Product Confidence Data

| Bit No. |  |
| --- | --- |
| 1 | No forebeam calculation |
| 2 | No midbeam calculation |
| 3 | No aftbeam calculation |
| 4 | Forebeam arcing detected |
| 5 | Midbeam arcing detected |
| 6 | Aftbeam arcing detected |
| 7 | Any beam noise content above or equal to threshold |
| 8 | Land (any land in cell footprint) |
| 9 | Autonomous ambiguity removal not used |
| 10 | Meteorological background not used |
| 11 | Minimum residual exceeded threshold |
| 12 | Frame checksum error detected |
| All 13 | Missing Value |

0 21 068

- Radar Altimeter Product Confidence Data

| Bit No. |  |
| --- | --- |
| 1 | Standard deviation wind speed outside MMCC limit |
| 2 | Standard deviation Significant wave height outside MMCC limit |
| 3 | Standard deviation altitude outside MMCC limit |
| 4 | Mean peakiness outside MMCC limit |
| 5 | Frame checksum error detected |
| 6 | Height-time loop time constant correction not performed |
| 7 | Not enough measurements (N<10) |
| All 8 | Missing Value |

Note:	MMCC is Mission Management Control Centre.

0 21 069

- SST product confidence data

| Bit No. |  |
| --- | --- |
| 1 | 12.0 m channel present in source data |
| 2 | 11.0 m channel present in source data |
| 3 | 3.7 m channel present in source data |
| 4 | 1.6 m channel present in source data |
| 5 | Cloud-identification used 1.6 um histogram reflectance cloud test |
| 6 | 1.6 m histogram reflectance cloud test used dynamic threshold |
| 7 | Sun glint detected by 1.6 um reflectance cloud test |
| 8 | 3.7 m channel used in sea-surface temperature retrieval |
| 9 | Sea-surface temperature derivation used day-time data (night-time if zero) |
| All 10 | Missing value |

0 21 070

SST product confidence data (SADIST-2)

| Bit | Meaning when set |  |
| --- | --- | --- |
| 1-9 | Nadir-only view SST retrieval used 3.7 micron channel (one bit per 10-arcmin cell) |  |
|  | 1 Cell 1:nadir-only view SST used 3.7 micron channel | Cell numbering NW NE 7 8 9 4 5 6 1 2 3 SW SE |
|  | 2 Cell 2:nadir-only view SST used 3.7 micron channel |  |
|  | 3 Cell 3:nadir-only view SST used 3.7 micron channel |  |
|  | 4 Cell 4:nadir-only view SST used 3.7 micron channel |  |
|  | 5 Cell 5:nadir-only view SST used 3.7 micron channel |  |
|  | 6 Cell 6:nadir-only view SST used 3.7 micron channel |  |
|  | 7 Cell 7:nadir-only view SST used 3.7 micron channel |  |
|  | 8 Cell 8:nadir-only view SST used 3.7 micron channel |  |
|  | 9 Cell 9:nadir-only view SST used 3.7 micron channel |  |
| 10-18 | Dual view SST retrieval used 3.7 micron channel (one bit per 10-arcmin cell) |  |
|  | 10 Cell 1:nadir-only view SST used 3.7 micron channel | Cell numbering NW NE 7 8 9 4 5 6 1 2 3 SW SE |
|  | 11 Cell 2:nadir-only view SST used 3.7 micron channel |  |
|  | 12 Cell 3:nadir-only view SST used 3.7 micron channel |  |
|  | 13 Cell 4:nadir-only view SST used 3.7 micron channel |  |
|  | 14 Cell 5:nadir-only view SST used 3.7 micron channel |  |
|  | 15 Cell 6:nadir-only view SST used 3.7 micron channel |  |
|  | 16 Cell 7:nadir-only view SST used 3.7 micron channel |  |
|  | 17 Cell 8:nadir-only view SST used 3.7 micron channel |  |
|  | 18 Cell 9:nadir-only view SST used 3.7 micron channel |  |
| 19 | Nadir view contains day-time data (night if zero) |  |
| 20 | Forward view contains day-time data (night if zero) |  |
| 21 | Record contains contributions from instrument scans acquired when ERS platform not in yaw-steering mode |  |
| 22 | Record contains contributions from instrument scans for which Product Confidence Data show quality is poor or unknown |  |
| All 23 | Missing value |  |

0 21 072

- Altimeter Calibration Status

| Bit No. |  |
| --- | --- |
| 1 | Height error correction applied instead of open loop calibration |
| 2 | Microwave sounder used for troposphere correction |
| 3 | AGC output correction applied instead of open loop calibration |
| All 4 | Missing value |

0 21 073

- Altimeter Instrument Mode

| Bit No. |  |
| --- | --- |
| 1 | Blank data record |
| 2 | Test |
| 3 | Calibration (closed loop) |
| 4 | BITE |
| 5 | Acquisition on ice |
| 6 | Acquisition on ocean |
| 7 | Tracking on ice |
| 8 | Tracking on ocean |
| All 9 | Missing value |

0 21 076

- Representation of intensities

| Code figure |  |
| --- | --- |
| 1 | Linear |
| 2 | Logarithmic (base e) |
| 3 | Logarithmic (base 10) |
| 3-6 | Reserved |
| 7 | Missing value |

0 21 109

SEAWINDS wind vector call quality

| Bit No. |  |
| --- | --- |
| 1 | Not enough good sigma-0 available for wind retrieval |
| 2 | Poor azimuth diversity among sigma-0 for wind retrieval |
| 3-7 | Reserved |
| 8 | Some portion of wind vector cell is over land |
| 9 | Some portion of wind vector cell is over ice |
| 10 | Wind retrieval not performed for wind vector cell |
| 11 | Reported wind speed is greater than 30 m s-1 |
| 12 | Reported wind speed is less than or equal to 3 m s-1 |
| 13-16 | Reserved |
| All 17 | Missing value |

0 21 115

SEAWINDS sigma-0 quality

| Bit No. |  |
| --- | --- |
| 1 | Sigma-0 measurement is not usable |
| 2 | Signal to noise ratio is low |
| 3 | Sigma-0 is negative |
| 4 | Sigma-0 is outside of acceptable range |
| 5 | Scattermeter pulse quality is not acceptable |
| 6 | Sigma-0 cell location algorithm does not converge |
| 7 | Frequency shift lies beyond the range of the x factor table |
| 8 | Spacecraft temperature is beyond calibration coefficient range |
| 9 | No applicable altitude records were found for this sigma-0 |
| 10 | Interpolated ephemeris data are not acceptable for this sigma-0 |
| 11-16 | Reserved |
| All 17 | Missing value |

0 21 116

SEAWINDS sigma-0 mode

| Bit No. |  |
| --- | --- |
| 1 | Calibration/measurement pulse flag (1) |
| 2 | Calibration/measurement pulse flag (2) |
| 3 | Outer antenna beam |
| 4 | Sigma-0 cell is aft of spacecraft |
| 5 | Current mode (1) |
| 6 | Current mode (2) |
| 7 | Effective gate width - slice resolution (1) |
| 8 | Effective gate width - slice resolution (2) |
| 9 | Effective gate width - slice resolution (3) |
| 10 | Low resolution mode - whole pulse data |
| 11 | Scatterometer electronic subsystem B |
| 12 | Alternate spin rate - 19.8 rpm |
| 13 | Receiver protection on |
| 14 | Slices per composite flag (1) |
| 15 | Slices per composite flag (2) |
| 16 | Slices per composite flag (3) |
| All 17 | Missing value |

0 21 119

Wind scatterometer geophysical model function

| Code figure |  |
| --- | --- |
| 0 | Reserved |
| 1 | SASS |
| 2 | SASS2 |
| 3 | NSCAT0 |
| 4 | NSCAT1 |
| 5 | NSCAT2 |
| 6 | QSCAT0 |
| 7 | QSCAT1 |
| 8-30 | Reserved |
| 31 | CMOD1 |
| 32 | CMOD2 |
| 33 | CMOD3 |
| 34 | CMOD4 |
| 35 | CMOD5 |
| 36-62 | Reserved |
| 63 | Missing value |

0 21 144

Altimeter rain flag

| Bit number |  |
| --- | --- |
| 1 | Rain |
| All 2 | Missing value |

0 22 056

- Direction of profile

| Code figure |  |
| --- | --- |
| 0 | Upwards profile |
| 1 | Downwards profile |
| 2 | Horizontal |
| 3 | Missing value |

0 22 060

Lagrangian drifter drogue status

| Code figure |  |
| --- | --- |
| 0 | Drogue is detached |
| 1 | Drogue is attached |
| 2 | Drogue status unknown |
| 3-6 | Reserved |
| 7 | Missing value |

0 22 061

- State of the sea

| Code figure | Descriptive terms | Height in metres |
| --- | --- | --- |
| 0 | Calm (glassy) | 0 |
| 1 | Calm (rippled) | 0-0.1 |
| 2 | Smooth (wavelets) | 0.1-0.5 |
| 3 | Slight | 0.5-1.25 |
| 4 | Moderate | 1.25-2.5 |
| 5 | Rough | 2.5-4 |
| 6 | Very rough | 4-6 |
| 7 | High | 6-9 |
| 8 | Very high | 9-14 |
| 9 | Phenomenal | Over 14 |
| 10-14 | Reserved |  |
| 15 | Missing value |  |

Notes:

- These values refer to well-developed wind waves of the open sea. While priority shall be given to the descriptive terms, these height values may be used for guidance by the observer when reporting the total state of agitation of the sea resulting from various factors such as wind, swell, currents, angle between swell and wind, etc.

(2) 	The exact bounding height shall be assigned for the lower code figure; e.g., a height of 4 m is coded as 5.

0 22 067

- Instrument type for water temperature profile measurement

(See common Code Table C-3)

0 22 068

- Water temperature profile recorder types

(See common Code Table C-4)

0 22 120

- Tide station automated water level check

| Code figure |  |
| --- | --- |
| 0 | Good data |
| 1 | Maximum (high) water level limit exceeded |
| 2 | Minimum (low) water level limit exceeded |
| 3 | Rate of change limit for water level exceeded |
| 4 | Flat limit for water level exceeded |
| 5 | Observed minus predicted water level value limit exceeded |
| 6 | Observed value from primary water level sensor minus backup water level sensor |
| 7 | Value exceeded specified tolerance from expected value |
| 8 | Water level QA parameter (sigmas and/or outliers) limits exceeded |
| 9 | Sea temperature outside of expected range |
| 10 | Multiple QC checks (above) failed |
| 11 | No automated water level checks performed |
| 12-30 | Reserved |
| 31 | Missing value |

0 22 121

- Tide station manual water level check

| Code figure |  |
| --- | --- |
| 0 | Operational |
| 1 | Possible clogging problem or otherwise degraded water level data |
| 2 | Possible datum shift |
| 3 | Unknown status of water level sensor |
| 4 | Suspected or known sea temperature sensor problem |
| 5 | Multiple possible problems (above) |
| 6 | Bad data - DO NOT DISSEMINATE! |
| 7 | No manual water level checks performed |
| 8-30 | Reserved |
| 31 | Missing value |

0 22 122

- Tide station automated meteorological data check

| Code figure |  |
| --- | --- |
| 0 | Good data from all sensors |
| 1 | Wind direction outside of allowable range |
| 2 | Wind speed outside of expected range |
| 3 | Barometric pressure outside of expected range |
| 4 | Air temperature outside of expected range |
| 5 | Multiple sensors failed QC checks |
| 6 | No automated meteorological data checks performed |
| 7-30 | Reserved |
| 31 | Missing value |

0 22 123

- Tide station manual meteorological data check

| Code figure |  |
| --- | --- |
| 0 | Operational |
| 1 | Suspected or known problem with wind sensor |
| 2 | Suspected or known problem with barometric pressure sensor |
| 3 | Suspected or known problem with air temperature sensor |
| 4 | Unknown status of all sensors |
| 5 | Suspected or known problems with multiple sensors |
| 6 | Bad data - DO NOT DISSEMINATE! |
| 7 | No manual meteorological data checks performed |
| 8-30 | Reserved |
| 31 | Missing value |

0 23 001

- Accident early notification - article applicable

| Code figure |  |
| --- | --- |
| 0 | Reserved |
| 1 | Articles 1 and 2 |
| 2 | Article 3 |
| 3 | Article 5.2 |
| 4-6 | Reserved |
| 7 | Missing value |

0 23 002

- Activity or facility involved In Incident

| Code figure |  |
| --- | --- |
| 0 | Reserved |
| 1 | Nuclear reactor on ground |
| 2 | Nuclear reactor at sea |
| 3 | Nuclear reactor In space |
| 4 | Nuclear fuel facility |
| 5 | Radioactive waste management facility |
| 6 | Transport of nuclear fuel or radioactive waste |
| 7 | Storage of nuclear fuel or radioactive waste |
| 8 | Manufacture of radio-isotopes |
| 9 | Use of radio-isotopes |
| 10 | Storage of radio-isotopes |
| 11 | Disposal of radio-isotopes |
| 12 | Transport of radio-isotopes |
| 13 | Use of radio-isotopes for power generation |
| 14-29 | Reserved |
| 30 | Other |
| 31 | Missing value |

0 23 003

- Type of release

| Code figure |  |
| --- | --- |
| 0 | No release |
| 1 | Release to atmosphere |
| 2 | Release to water |
| 3 | Release to both atmosphere and water |
| 4 | Expected release to atmosphere |
| 5 | Expected release to water |
| 6 | Expected release to both atmosphere and water |
| 7 | Missing value |

0 23 004

- Countermeasures taken near border

| Code figure |  |
| --- | --- |
| 0 | No countermeasures |
| 1 | Evacuation |
| 2 | Sheltering |
| 3 | Prophylaxis |
| 4 | Water |
| 5-6 | Reserved |
| 7 | Missing value |

0 23 005

- Cause of incident

| Code figure |  |
| --- | --- |
| 0 | Incident State does not understand what happened |
| 1 | Incident State knows the cause of the incident |
| 2 | Reserved |
| 3 | Missing value |

0 23 006

- Incident situation

| Code figure |  |
| --- | --- |
| 0 | No improvement |
| 1 | Unstable |
| 2 | No deterioration |
| 3 | Improving |
| 4 | Stable |
| 5 | Deteriorating |
| 6 | Reserved |
| 7 | Missing value |

0 23 007

- Characteristics of release

| Code figure |  |
| --- | --- |
| 0 | No release |
| 1 | Release has stopped |
| 2 | Release |
| 3 | Release is continuing |
| 4-6 | Reserved |
| 7 | Missing value |

0 23 008 / 0 23 009

- State of current or expected release

| Code figure |  |
| --- | --- |
| 0 | Gaseous |
| 1 | Particulate |
| 2 | Mixture of gaseous and particulate |
| 3 | Missing value |

0 23 016

- Possibility of significant chemical toxic health effect

| Code figure |  |
| --- | --- |
| 0 | No significant chemical toxic health effect |
| 1 | Significant chemical toxic health effect possible |
| 2 | Reserved |
| 3 | Missing value |

0 23 018

- Release behaviour over time

| Code figure |  |
| --- | --- |
| 0 | Release no longer occurring |
| 1 | Release still occurring |
| 2 | Release expected to increase in next 6 hours |
| 3 | Release expected to remain constant in next 6 hours |
| 4 | Release expected to decrease in next 6 hours |
| 5-6 | Reserved |
| 7 | Missing value |

0 23 031

- Possibility that plume will encounter precipitation in State in which incident occurred

| Code figure |  |
| --- | --- |
| 0 | Plume will not encounter rain in incident State |
| 1 | Plume will encounter rain in incident State |
| 2 | Reserved |
| 3 | Missing value |

0 23 032

- Plume will encounter change in wind direction and/or speed flag

| Code figure |  |
| --- | --- |
| 0 | No significant change expected within the next 6 hours |
| 1 | Anticipated significant change expected within the next 6 hours |
| 2 | Reserved |
| 3 | Missing value |

0 24 003

- Composition of release

| Code figure |  |
| --- | --- |
| 0 | Noble gases |
| 1 | Iodides |
| 2 | Caesiums |
| 3 | Transuranics |
| 4-30 | Reserved |
| 31 | Missing value |

0 25 004

- Echo processing

| Code figure |  |
| --- | --- |
| 0 | Incoherent |
| 1 | Coherent (Doppler) |
| 2 | Reserved |
| 3 | Missing value |

0 25 005

- Echo Integration

| Code figure |  |
| --- | --- |
| 0 | Logarithm - 2.5dB |
| 1 | Linear |
| 2 | Special |
| 3 | Missing value |

0 25 006

- Z to R conversion

| Code figure |  |
| --- | --- |
| 0 | ZH to R conversion |
| 1 | (ZH, ZDR) to (NO, DO) to R |
| 2 | (Z (Fl), Z (F2)) to attenuation to R |
| 3-5 | Reserved |
| 6 | Other |
| 7 | Missing value |

0 25 009

- Calibration method

| Bit No. |  |
| --- | --- |
| 1 | None |
| 2 | Calibration target or signal |
| 3 | Against rain gauges |
| 4 | Against other Instruments (distrometer - attenuation) |
| All 4 | Missing value |

0 25 010

- Clutter treatment

| Code figure |  |
| --- | --- |
| O | None |
| 1 | Map |
| 2 | Insertion of higher elevation data and map |
| 3 | Analysis of the fluctuating Logarithm signal (clutter detection) |
| 4 | Extraction of the fluctuating part of linear signal (clutter suppression) |
| 5 | Clutter suppression - Doppler |
| 6 | Multi-parameter analysis |
| 7-14 | Reserved |
| 15 | Missing value |

0 25 011

- Ground occultation correction (screening)

| Code figure |  |
| --- | --- |
| 0 | None |
| 1 | Map of correction factors |
| 2 | Interpolation (azimuth or elevation) |
| 3 | Missing value |

0 25 012

- Range Attenuation Correction

| Code figure |  |
| --- | --- |
| 0 | Hardware |
| 1 | Software |
| 2 | Hardware and software |
| 3 | Missing value |

0 25 013

- Bright-band correction

| Bit No. |  |
| --- | --- |
| 1 | Brightband correction |
| all 2 | Missing value |

0 25 015

- Radome Attenuation Correction

| Bit No. |  |
| --- | --- |
| 1 | Radome Attenuation Correction |
| All 2 | Missing value |

0 25 017

- Precipitation attenuation correction

| Bit No. |  |
| --- | --- |
| 1 | Precipitation attenuation correction |
| All 2 | Missing value |

0 25 020

- Mean-speed estimation

| Code figure |  |
| --- | --- |
| 0 | FFT (fast Fourier transform) |
| 1 | PP (pulse-pair processing) |
| 2 | VPC (vector-phase change) |
| 3 | Missing value |

0 25 021

- Wind computation enhancement

| Bit No. |  |
| --- | --- |
| 1 | Simple average |
| 2 | Consensus average |
| 3 | Median check |
| 4 | Vertical consistency check |
| 5 | Other |
| 6-7 | Reserved |
| All 8 | Missing value |

0 25 030

- Running mean sea-surface temperature usage

| Code figure |  |
| --- | --- |
| 0 | Running mean sea-surface temperature not used because usage criteria not met |
| 1 | Running mean sea-surface temperature not used because data not available |
| 2 | Running mean sea-surface temperature used as predictor |
| 3 | Missing value |

0 25 032

- NOAA wind profiler mode information

| Code figure |  |
| --- | --- |
| 0 | Reserved |
| 1 | Data from low mode |
| 2 | Data from high mode |
| 3 | Missing value |

0 25 033

- NOAA wind profiler submode information

| Code figure |  |
| --- | --- |
| 0 | Wind Profiler operating in Submode A |
| 1 | Wind Profiler operating in Submode B |
| 2 | Reserved |
| 3 | Missing value |

0 25 034

- NOAA wind profiler quality control test results

| Bit No. | Meaning (1=true, 0=false) |
| --- | --- |
| 1 | Test A performed and failed |
| 2 | Test B performed and failed |
| 3 | Test results inconclusive |
| All 4 | Missing value |

0 25 036

Atmospherics location method

| Code figure |  |
| --- | --- |
| 0 | Network of several direction-finders operating on the same individual atmospherics |
| 1 | Network of several arrival-time stations operating on the same individual atmospherics |
| 2-5 | Reserved |
| 6 | Single station range bearing technique |
| 7-14 | Reserved |
| 15 | Missing value |

0 25 040

- CO2 Wind Product Derivation

| Code figure |  |
| --- | --- |
| 0 | Non-specific mode |
| 1 | First guess data |
| 2 | Cloud data |
| 3 | Average vector data |
| 4 | Primary data |
| 5 | Guess data |
| 6 | Vector data |
| 7 | Tracer data;this image |
| 8 | Tracer data to next image |
| 9-14 | Reserved |
| 15 | Missing value |

0 25 041

- Moving platform direction reporting method

| Code figure |  |
| --- | --- |
| 0 | Direction originally reported in true degrees |
| 1 | Direction originally reported using Code Table 0700, FM13 |
| 2 | Reserved |
| 3 | Missing value |

NOTE:	Where the original reporting method is as indicated by code figure 1, the following conversion is reccommended to obtain a suitable data value corresponding to descriptor 0 01 012:

| Reported value | Data value |
| --- | --- |
| 0 | 0 |
| 1 | 45 |
| 2 | 90 |
| 3 | 135 |
| 4 | 180 |
| 5 | 225 |
| 6 | 270 |
| 7 | 315 |
| 8 | 360 |
| 9 | 511 |

0 25 042

- Moving platform speed reporting method

| Code figure |  |
| --- | --- |
| 0 | Speed originally reported in metres per second |
| 1 | Speed originally reported using Code Table 4451, FM13 |
| 2 | Reserved |
| 3 | Missing value |

NOTE:	Where the original reporting method is as indicated by code figure 1, the following conversion is recommended to obtain a suitable data value corresponding to descriptor 0 01 013:

| Reported value | Data value |
| --- | --- |
| 0 | 0 |
| 1 | 1 |
| 2 | 4 |
| 3 | 7 |
| 4 | 9 |
| 5 | 12 |
| 6 | 14 |
| 7 | 17 |
| 8 | 19 |
| 9 | 21 |
| / | 1023 |

0 25 045

- HIRS channel combination

| Bit No. |  |
| --- | --- |
| 1-20 | Beginning with first bit position (high order bit), if bit position is set to 1, then channel is present, if bit position is set to 0, then channel is not present |
| All 21 | Missing value |

0 25 046

- MSU channel combination

| Bit No. |  |
| --- | --- |
| 1-4 | Beginning with first bit position (high order bit), if bit position is set to 1, then channel is present, if bit position is set to 0, then channel is not present |
| All 5 | Missing value |

0 25 047

- SSU channel combination

| Bit No. |  |
| --- | --- |
| 1-3 | Beginning with first bit position (high order bit); if bit position is set to 1, then channel is present; if bit position is set to 0, then channel is not present |
| All 4 | Missing value |

0 25 048

- AMSU-A channel combination

| Bit No. |  |
| --- | --- |
| 1-15 | Beginning with first bit position (high order bit), if bit position is set to 1, then channel is present, if bit position is set to 0, then channel is not present |
| All 16 | Missing value |

0 25 049

- AMSU-B channel combination

| Bit No. |  |
| --- | --- |
| 1-5 | Beginning with first bit position (high order bit), if bit position is set to 1, then channel is present, if bit position is set to 0, then channel is not present |
| All 6 | Missing value |

0 25 051

- AVHRR channel combination

| Bit No. |  |
| --- | --- |
| 1-6 | Beginning with first bit position (high order bit), if bit position is set to 1, then channel is present, if bit position is set to 0, then channel is not present |
| All 7 | Missing value |

0 25 053

- Observation quality

| Bit No. |  |
| --- | --- |
| 1 | Good |
| 2 | Redundant |
| 3 | Questionable |
| 4 | Bad |
| 5 | Experimental |
| 6 | Precipitating |
| 7-11 | Reserved |
| All 12 | Missing value |

0 25 069

Flight Level Pressure Corrections

| Bit No. |  |
| --- | --- |
| 1 | Smoothed |
| 2 | Baseline adjusted |
| 3 | Normalized time interval |
| 4 | Outlier checked |
| 5 | Plausibility checked |
| 6 | Consistency checked |
| 7 | Interpolated |
| All 8 | Missing value |

0 25 086

Depth correction indicator

| Code figure |  |
| --- | --- |
| 0 | Depths are not corrected |
| 1 | Depths are corrected |
| 2 | Reserved |
| 3 | Missing value |

0 25 093

RASS computation correction

| Bit No. |  |
| --- | --- |
| 1 | No correction |
| 2 | Vertical velocity correction |
| 3-6 | Reserved |
| 7 | All corrections |
| All 8 | Missing value |

0 25 095

# Altimeter state flag

| Bit No. |  |
| --- | --- |
| 1 | Altimeter operating (set to 0 if nominal, set to 1 if backup) |
| All 2 | Missing value |

0 25 096

# Radiometer state flag

| Bit No. |  |
| --- | --- |
| 1 | Mode indicator (0 if Mode 2, 1 if Mode 1) |
| 2 | Mode 1 Calibration sequence indicator (0 if normal data taking either Mode 1 or 2, 1 if Mode 1 Calibration sequence) |
|  | Bits 3 and 4 indicate active 23.8 GHz channel |
| 3 | Channel 2 (0 if on, 1 if off) |
| 4 | Channel 3 (0 if on, 1 if off) |
| All 5 | Missing value |

0 25 097

Three-dimensional error estimate of the navigator orbit

| Code figure |  |
| --- | --- |
|  | Ranges between 0 and 30 cm |
|  | Ranges between 30 and 60 cm |
|  | Ranges between 60 and 90 cm |
|  | Ranges between 90 and 120 cm |
|  | Ranges between 120 and 150 cm |
|  | Ranges between 150 and 180 cm |
|  | Ranges between 180 and 210 cm |
|  | Ranges between 210 and 240 cm |
|  | Ranges between 240 and 270 cm |
|  | Ranges larger than 270 cm |
| 10-14 | Reserved |
| 15 | Missing value |

0 25 110

# Image processing summary

| Bit No. |  |
| --- | --- |
| 1 | Raw data analysis used for raw data correction. Correction done using default parameters |
| 2 | Raw data analysis used for raw data correction. Correction done using raw data analysis results |
| 3 | Antenna elevation pattern correction applied |
| 4 | Nominal chirp replica used |
| 5 | Reconstructed chirp used |
| 6 | Slant range to ground range conversion applied |
| 7-9 | Reserved |
| All 10 | Missing value |

0 25 120

RA2_l2_processing flag

| Code figure |  |
| --- | --- |
| 0 | Percentage of DSRs free of processing errors during Level 2 processing is greater than the acceptable threshold |
| 1 | Percentage of DSRs free of processing errors during Level 2 processing is less than the acceptable threshold |
| 2 | Reserved |
| 3 | Missing value |

Note: DSR = Data Set Record

0 25 122

Hardware configuration for RF

| Code figure |  |
| --- | --- |
| 0 | Hardware configuration for RF is A |
| 1 | Hardware configuration for RF is B |
| 2 | Reserved |
| 3 | Missing value |

Note: RF = Radio Frequency

0 25 123

Hardware configuration for HPA

| Code figure |  |
| --- | --- |
| 0 | Hardware configuration for HPA is A |
| 1 | Hardware configuration for HPA is B |
| 2 | Reserved |
| 3 | Missing value |

Note: HPA = High Power Amplifier

0 25 124

MWR l2 processing flag

| Code figure |  |
| --- | --- |
| 0 | Percentage of DSRs free of processing errors during Level 2 processing is greater than the acceptable threshold |
| 1 | Percentage of DSRs free of processing errors during Level 2 processing is less than the acceptable threshold |
| 2 | Reserved |
| 3 | Missing value |

Note:	DSR = Data Set Record

MWR = Microwave radiometer

0 26 010

- Hours included

| Bit No. |  |
| --- | --- |
| 1 | 0100 included |
| 2 | 0200 included |
| 3 | 0300 included |
| 4 | 0400 included |
| 5 | 0500 included |
| 6 | 0600 included |
| 7 | 0700 included |
| 8 | 0800 included |
| 9 | 0900 included |
| 10 | 1000 included |
| 11 | 1100 included |
| 12 | 1200 included |
| 13 | 1300 included |
| 14 | 1400 included |
| 15 | 1500 included |
| 16 | 1600 included |
| 17 | 1700 included |
| 18 | 1800 included |
| 19 | 1900 included |
| 20 | 2000 included |
| 21 | 2100 included |
| 22 | 2200 included |
| 23 | 2300 included |
| 24 | 2400 included |
| 25 | Unknown mixture of hours |
| All 26 | Missing value |

0 29 001

- Projection type

| Code figure |  |
| --- | --- |
| 0 | Gnomonic projection |
| 1 | Polar stereographic projection |
| 2 | Lambert's conformal conic projection |
| 3 | Mercator's projection |
| 4 | Scanning Cone (radar)* |
| 5-6 | Reserved |
| 7 | Missing value |

*	Note: Projection type 4 indicates a Cartesian grid placed directly on the scanning cone defined by the azimuthal sweep of the radar.

0 29 002

- Co-ordinate grid type

| Code figure |  |
| --- | --- |
| 0 | Cartesian |
| 1 | Polar |
| 2 | Other |
| 3-6 | Reserved |
| 7 | Missing value |

0 30 031

- Picture type

| Code figure |  |
| --- | --- |
| 0 | PPI |
| 1 | Composite |
| 2 | CAPPl |
| 3 | Vertical section |
| 4 | Alphanumeric data |
| 5 | Map of subject clutter |
| 6 | Map |
| 7 | Test picture |
| 8 | Comments |
| 9 | Map of ground occultation |
| 10 | Map of radar beam height |
| 11-13 | Reserved |
| 14 | Other |
| 15 | Missing value |

0 30 032

- Combination with other data

| Bit No. |  |
| --- | --- |
| 1 | Map |
| 2 | Satellite IR |
| 3 | Satellite VIS |
| 4 | Satellite WV |
| 5 | Satellite multispectral |
| 6 | Synoptic observations |
| 7 | Forecast parameters |
| 8 | Lightning data |
| 9-14 | Reserved |
| 15 | Other data |
| All 16 | Missing value |

0 31 021

- Associated field significance

| Code figure |  |  |
| --- | --- | --- |
| 0 | Reserved |  |
| 1 | 1-bit indicator of quality | 0 = good 1 = suspect or bad |
| 2 | 2-bit indicator of quality | 0 = good 1 = slightly suspect 2 = highly suspect 3 = bad |
| 3-5 | Reserved |  |
| 6 | Quality Control Flag according to GTSPP | 0 = Unqualified 1 = Correct value (all checks passed) 2 = Probably good but value inconsistent with statistics (differ from climatology) 3 = Probably bad (spike, gradient, … if other tests passed) 4 = Bad value, Impossible value (out of scale, vertical instability, constant profile) 5 = Value modified during quality control 6-7 = Not used (reserved) 8 = Interpolated value 9 = Missing value |
| 7 | Percentage confidence |  |
| 8-20 | Reserved |  |
| 21 | 1-bit indicator of correction (see Note (2)) | 0 = original value 1 = substituted/corrected value |
| 22-62 | Reserved for local use |  |
| 63 | Missing value |  |

- N o t e s

(1)	Associated field significance shall be used initially in conjunction with the quality of observed data.

(2)	The code figure 21 may be used within corrected messages with the corrected/ substituted values identified.

(3)	Further applications may be developed.

0 31 031

- Data Present Indicator

| Bit No. | Value | Meaning |
| --- | --- | --- |
| 1 | 0 | Data present |
|  | 1 | Data not present |

0 33 002

- Quality Information

| Code figure |  |
| --- | --- |
| 0 | Data not suspect |
| 1 | Data suspect |
| 2 | Reserved |
| 3 | Quality information not given |

0 33 003

- Quality Information

| Code figure |  |
| --- | --- |
| 0 | Data not suspect |
| 1 | Data slightly suspect |
| 2 | Data highly suspect |
| 3 | Data considered unfit for use |
| 4-6 | Reserved |
| 7 | Quality information not given |

0 33 005

Quality Information (AWS data)

| Bit No. |  |
| --- | --- |
| 1 | No automated meteorological data checks performed |
| 2 | Pressure data suspect |
| 3 | Wind data suspect |
| 4 | Dry-bulb temperature data suspect |
| 5 | Wet-bulb temperature data suspect |
| 6 | Humidity data suspect |
| 7 | Ground temperature data suspect |
| 8 | Soil temperature (depth 1) data suspect |
| 9 | Soil temperature (depth 2) data suspect |
| 10 | Soil temperature (depth 3) data suspect |
| 11 | Soil temperature (depth 4) data suspect |
| 12 | Soil temperature (depth 5) data suspect |
| 13 | Cloud data suspect |
| 14 | Visibility data suspect |
| 15 | Present weather data suspect |
| 16 | Lightning data suspect |
| 17 | Ice deposit data suspect |
| 18 | Precipitation data suspect |
| 19 | State of ground data suspect |
| 20 | Snow data suspect |
| 21 | Water content data suspect |
| 22 | Evaporation/evapotranspiration data suspect |
| 23 | Sunshine data suspect |
| 24-29 | Reserved |
| All 30 | Missing value |

0 33 006

Internal measurement status information (AWS)

| Code figure |  |
| --- | --- |
| 0 | Self-check OK |
| 1 | At least one Warning active, no Alarms |
| 2 | At least one Alarm active |
| 3 | Sensor failure |
| 4-6 | Reserved |
| 7 | Missing value |

0 33 015

## Data Quality Check Indicator

| Code figure |  |
| --- | --- |
| 0 | Passed all checks |
| 1 | Missing-data check |
| 2 | Descending/reascending balloon check |
| 3 | Data plausibility check (above limits) |
| 4 | Data plausibility check (below limits) |
| 5 | Superadiabatic lapse rate check |
| 6 | Limiting angles check |
| 7 | Ascension rate check |
| 8 | Excessive change from previous flight |
| 9 | Balloon overhead check |
| 10 | Wind speed check |
| 11 | Wind direction check |
| 12 | Dependency check |
| 13 | Data valid but modified |
| 14 | Data outlier check |
| 15-62 | Reserved |
| 63 | Missing value |

0 33 020

- Quality control indication of following value

| Code figure |  |
| --- | --- |
| 0 | Good |
| 1 | Inconsistent |
| 2 | Doubtful |
| 3 | Wrong |
| 4 | Not checked |
| 5 | Has been changed |
| 6 | Estimated |
| 7 | Missing value |

0 33 021

- Quality of following value

| Code figure |  |
| --- | --- |
| 0 | Within limits |
| 1 | Outside limits |
| 2 | Reserved |
| 3 | Missing value |

0 33 022

- Quality of buoy satellite transmission

| Code figure |  |
| --- | --- |
| 0 | Good (several identical reports have been received) |
| 1 | Dubious (no identical reports have been received) |
| 2 | Reserved |
| 3 | Missing value |

0 33 023

- Quality of buoy location

| Code figure |  |
| --- | --- |
| 0 | Reliable (location was made over two satellite passes) |
| 1 | Latest known (no location over the corresponding pass) |
| 2 | Dubious (location made over one pass only; a second solution is possible in 5 per cent of the cases) |
| 3 | Missing value |

0 33 024

- Station elevation quality mark (for mobile stations)

| Code figure |  |
| --- | --- |
| 0 | Reserved |
| 1 | Excellent - within 3 meters |
| 2 | Good - within 10 meters |
| 3 | Fair - within 20 meters |
| 4 | Poor - more than 20 meters |
| 5 | Excellent - within 10 feet |
| 6 | Good - within 30 feet |
| 7 | Fair - within 60 feet |
| 8 | Poor - more than 60 feet |
| 9-14 | Reserved |
| 15 | Missing value |

0 33 025

- ACARS interpolated values

| Code figure |  |
| --- | --- |
| 0 | Time interpolated, latitude and longitude reported |
| 1 | Time reported, latitude and longitude interpolated |
| 2 | Time, latitude, and longitude interpolated |
| 3 | Time, latitude, and longitude reported |
| 4-6 | Reserved |
| 7 | Missing value |

0 33 026

- Moisture quality

| Code figure |  |
| --- | --- |
| 0 | Normal operations - measurement mode |
| 1 | Normal operations - non-measurement mode |
| 2 | Small RH |
| 3 | Humidity element is wet |
| 4 | Humidity element contaminated |
| 5 | Heater fail |
| 6 | Heater fail and wet/contaminated humidity element |
| 7 | At least one of the input parameters used in the calculation of mixing ratio is invalid |
| 8 | Numeric error |
| 9 | Sensor not installed |
| 10-62 | Reserved |
| 63 | Missing value |

- 0 33 027

- Location quality class (range of radius of 66% confidence)

| Code figure |  |
| --- | --- |
| 0 | Radius > 1500 m |
| 1 | 500 m < Radius <1500 m |
| 2 | 250 m < Radius < 500 m |
| 3 | Radius < 250 m |
| 4-6 | Reserved |
| 7 | Missing value |

0 33 030

- Scan line status flags for ATOVS

| Bit No. |  |
| --- | --- |
| 1 | Do not use scan for product generation |
| 2 | Time sequence error detected with this scan |
| 3 | Data gap precedes this scan |
| 4 | No calibration |
| 5 | No earth location |
| 6 | First good time following a clock update |
| 7 | Instrument status changed with this scan |
| 8-23 | Reserved |
| All 24 | Missing value |

- Notes:	If bit is set to 1 then statement is true.

0 33 031

- Scan line quality flags for ATOVS

| Bit No. |  |
| --- | --- |
| 1 | Time field is bad but can probably be inferred from the previous good time |
| 2 | Time field is bad and can't be inferred from the previous good time |
| 3 | This record starts a sequence that is inconsistent with previous times (i.e. there is a time discontinuity). This may or may not be associated with a spacecraft clock update (see scan line status flags for ATOVS) |
| 4 | Start of a sequence that apparently repeats scan times that have been previously accepted |
| 5 | Scan line was not calibrated because of bad time |
| 6 | Scan line was calibrated using fewer than the preferred number of scan lines because of proximaty to start or end of data or to a data gap |
| 7 | Scan line was not calibrated because of bad or insufficient PRT data |
| 8 | Scan line was calibrated but with marginal PRT data |
| 9 | Some uncalibrated channels on this scan |
| 10 | Uncalibrated due to instrument mode |
| 11 | Questionable calibration because of antenna position error of space view |
| 12 | Questionable calibration because of antenna position error of black body |
| 13 | Not earth located because of bad time |
| 14 | Earth location questionable because of questionable time code (see time problem code bits) |
| 15 | Earth location questionable - only marginal agreement with reasonableness check |
| 16 | Earth location questionable - fails reasonableness check |
| 17 | Earth location questionable because of antenna position check |
| 18 | Scan line calibration cold black body |
| 19 | Scan line calibration warm black body |
| 20 | Scan line calibration space view |
| 21 | Earth view |
| 22-23 | Reserved |
| All 24 | Missing value |

Notes:

(1)	If bit is set to 1 then statement is true.

(2)	Bits 1-4 represent TIME PROBLEM CODE. All bits off implies the scan time is as expected.

(3)	Bits 5-10 represent CALIBRATION PROBLEM CODE. All bits set to zero indicates normal calibration. Where any of bits 5,7,10 are set, secondary calibration coefficients have been used.

(4)	Bits 11-17 represent EARTH LOCATION PROBLEM CODE. All bits set to zero implies the earth location was normal.

0 33 032

- Channel quality flags for ATOVS

| Bit No. |  |
| --- | --- |
| 1 | No good blackbody counts for scan line |
| 2 | No good space view counts for this line |
| 3 | No good PRTs for this line |
| 4 | Some bad blackbody view counts for this line |
| 5 | Some bad space view counts for this line |
| 6 | Some bad PRT temps on this line |
| 7-23 | Reserved (bits set to zero) |
| All 24 | Missing value |

- Notes:	All bits off implies a good calibration

0 33 033

- Field of view quality flags for ATOVS

| Bit No. |  |
| --- | --- |
| 1 | Set if secondary calibration used |
| 2-21 | Bit n set to 1 if brightness temperature in channel n-1 is physically unreasonable or has not been calculated due to calibration problems |
| 22 | Set if all the channels are missing |
| 23 | Suspect |
| All 24 | Missing value |

Notes:

(1)	All bits off implies a good calibration

(2)	Bits 2-21 used for HIRS, but only bits 2-16 used for AMSU-A and only bits 2-6 used for AMSU-B

0 33 035

- Manual/automatic quality control

| Code figure |  |
| --- | --- |
| 0 | Automatic quality control passed and not manually checked |
| 1 | Automatic quality control passed and manually checked and passed |
| 2 | Automatic quality control passed and manually checked and deleted |
| 3 | Automatic quality control failed and manually not checked |
| 4 | Automatic quality control failed and manually checked and failed |
| 5 | Automatic quality control failed and manually checked and re-inserted |
| 6 | Automatic quality control flagged data as questionable and not manually checked |
| 7 | Automatic quality control flagged data as questionable and manually checked and failed |
| 8 | Manually checked and failed |
| 9-14 | Reserved |
| 15 | Missing value |

0 33 037

- Wind correlation error

| Bit No. |  |
| --- | --- |
| 1 | U departure from guess |
| 2 | V departure from guess |
| 3 | U & V departure from guess |
| 4 | U acceleration |
| 5 | V acceleration |
| 6 | U & V acceleration |
| 7 | Possible land feature |
| 8 | U acceleration and possible land feature |
| 9 | V acceleration and possible land feature |
| 10 | U & V acceleration and possible land feature |
| 11 | Bad wind guess |
| 12 | Correlation failure |
| 13 | Search box off edge of area |
| 14 | Target box off edge of area |
| 15 | Pixel brightness out of bounds (noisy line) |
| 16 | Target outside of lat/long box |
| 17 | Target outside of pressure min/max |
| 18 | Autoeditor flagged slow vector |
| 19 | Autoeditor flagged vectors |
| All 20 | Missing value |

0 33 038

Quality Flags for ground-based GNSS data

| Bit No. |  |
| --- | --- |
| 1 | Total Zenith Delay quality is considered poor |
| 2 | GALILEO satellites used |
| 3 | GLONASS satellites used |
| 4 | GPS satellites used |
| 5 | Meteorological data applied |
| 6 | Atmospheric loading correction applied |
| 7 | Ocean tide loading applied |
| 8 | Climate quality data processing |
| 9 | Near-real time data processing |
| All 10 | Missing value |

0 33 039

Quality flags for Radio Occultation data

| Bit No. |  |
| --- | --- |
| 1 | Non-nominal quality |
| 2 | Offline product |
| 3 | Ascending occultation flag |
| 4 | Excess Phase processing non-nominal |
| 5 | Bending Angle processing non-nominal |
| 6 | Refractivity processing non-nominal |
| 7 | Meteorological processing non-nominal |
| 8-13 | Reserved |
| 14 | Background profile non-nominal |
| 15 | Background (i.e. not retrieved) profile present |
| All 16 | Missing value |

0 33 041

Attribute of following value

| Code figure |  |
| --- | --- |
| 0 | The following value is the true value |
| 1 | The following value is higher than the true value (the measurement hit the lower limit of the instrument) |
| 2 | The following value is lower than the true value (the measurement hit the higher limit of the instrument) |
| 3 | Missing value |

Note:	This descriptor will be associated with visibility data or height of clouds data to specify if the value is bounded.  If the reported data is the true value, the code figure is 0. However, the measurement can hit the limit of the instrument measurement capability. If the reported value is higher than the true value, the code figure is 1, if the reported value is lower than the true value, the code figure is 2.

0 33 042

- Type of limit represented by following value

| Code figure |  |
| --- | --- |
| 0 | Exclusive lower limit (>) |
| 1 | Inclusive lower limit (>=) |
| 2 | Exclusive upper limit (<) |
| 3 | Inclusive upper limit (=<) |
| 4-6 | Reserved |
| 7 | Missing value |

0 33 043

Flag table AST confidence

| Bit No. |  |
| --- | --- |
| 1 | Sea MDS. Nadir only SST retrieval used 3.7 |
| 2 | Micron channel. Land MDS reserved |
| 3 | Nadir view contains day time data |
| 4 | Forward view contains day time data |
| 5-7 | Reserved |
| All 8 | Missing value |

0 33 0044

ASAR quality information

| Bit No. |  |
| --- | --- |
| 1 | Input data mean outside nominal range flag |
| 2 | Input data standard deviation outside nominal range flag |
| 3 | Number of input data gaps > threshold value |
| 4 | Percentage of missing lines > threshold value |
| 5 | Doppler centroid uncertain. Confidence measure < specific value |
| 6 | Doppler ambiguity estimate uncertain. Confidence measure < specific value |
| 7 | Output data mean outside nominal range flag |
| 8 | Output data standard deviation outside nominal range flag |
| 9 | Chirp reconstruction failed or is of low quality flag |
| 10 | Data set missing |
| 11 | Invalid downlink parameters |
| 12 | Azimuth cut-off iteration count. The azimuth cut-off fit did not converge within a minimum number of iterations |
| 13 | Azimuth cut-off fit did not converge within a minimum number of iterations |
| 14 | Phase information confidence measure. The imaginary spectral peak is less than a minimum threshold, or the zero lag shift is greater than a minimum threshold |
| All 15 | Missing value |

0 33 0047

Measurement confidence data

| Bit No. |  |
| --- | --- |
| 1 | Error detected and attempts to recover made |
| 2 | Anomaly in on-board data handling (OBDH) value detected |
| 3 | Anomaly in Ultra Stable Oscillator Processing (USOP) value detected |
| 4 | Errors detected by on-board computer |
| 5 | Automatic gain control (AGC) out of range |
| 6 | Rx delay fault. Rx distance out of range |
| 7 | Wave form samples fault identifier. Error |
| 8-11 | Reserved |
| 12 | Brightness temperature (channel 1) out of range |
| 13 | Brightness temperature (channel 2) out of range |
| 14 | Reserved |
| 15 | Ku Ocean retracking error |
| 16 | S Ocean retracking error |
| 17 | Ku Ice 1 retracking error |
| 18 | S Ice 1 retracking error |
| 19 | Ku Ice 2 retracking error |
| 20 | S Ice 2 retracking error |
| 21 | Ku Sea Ice retracking error |
| 22 | Arithmetic fault error |
| 23 | Meteo data state. No map |
| 24 | Meteo data state. 1 map |
| 25 | Meteo data state 2 maps degraded |
| 26 | Meteo data state 2 maps nominal |
| 27 | Orbit propagator status for propagation mode, several errors |
| 28 | Orbit propagator status for propagation mode, warning detected |
| 29 | Orbit propagator status for initialisation mode, several errors |
| 30 | Orbit propagator status for initialisation mode, warning detected |
| All 31 | Missing value |

0 33 048

- Confidence measure of SAR inversion

| Code figure |  |
| --- | --- |
| 0 | Inversion successful |
| 1 | Inversion not successful |
| 2 | Reserved |
| 3 | Missing value |

0 33 049

- Confidence measure of wind retrieval

| Code figure |  |
| --- | --- |
| 0 | External wind direction used during inversion |
| 1 | External wind direction not used during inversion |
| 2 | Reserved |
| 3 | Missing value |

0 33 050

- Global GTSPP quality flag

| Code figure |  |
| --- | --- |
| 0 | Unqualified |
| 1 | Correct value (all checks passed) |
| 2 | Probably good but value inconsistent with statistics (differ from climatology) |
| 3 | Probably bad (spike, gradient, … if other tests passed) |
| 4 | Bad value, Impossible value (out of scale, vertical instability, constant profile) |
| 5 | Value modified during quality control |
| 6-7 | Reserved |
| 8 | Interpolated value |
| 9-14 | Reserved |
| 15 | Missing value |

0 33 052

S band ocean retracking quality

| Bit No. |  |
| --- | --- |
| 1-20 | First 20 least significant bits correspond to the 20 values (one per data block containing: 0=valid measurement, 1=invalid). Bit 1 applies to the 20th data block. |
| All 21 | Missing value |

0 33 053

Ku band ocean retracking quality

| Bit No. |  |
| --- | --- |
| 1-20 | First 20 least significant bits correspond to the 20 values (one per data block containing: 0=valid measurement, 1=invalid). Bit 1 applies to the 20th data block. |
| All 21 | Missing value |

0 35 000

FM and Regional Code number

| Code figure |  |
| --- | --- |
| 000-099 | International FM Codes |
| 100-199 | RA I Codes |
| 200-299 | RA II Codes |
| 300-399 | RA III Codes |
| 400-499 | RA IV Codes |
| 500-599 | RA V Codes |
| 600-699 | RA VI Codes |
| 700-799 | Antarctic Codes |
| 800-999 | Reserved |
| 1000-1022 | Not used |
| 1023 | Missing value |

0 35 001

Time-frame for monitoring

| Code figure |  |
| --- | --- |
| 0 | Real time |
| 1 | Near-real time |
| 2 | Non-real time |
| 3 | Reserved |
| 4 | Reserved |
| 5 | Reserved |
| 6 | Reserved |
| 7 | Missing value |

0 35 030

Discrepancies in the availability of expected data

| Code figure |  |
| --- | --- |
| 0 | No discrepancies |
| 1 | Non-compliance with standard and recommended practices and procedures including those of monitoring |
| 2 | Catalogues of meteorological bulletins not updated in a timely manner |
| 3 | Incorrect routeing directories |
| 4 | Lack of flexibility in the routeing arrangements |
| 5 | Deficiencies in the operation of GTS centres and circuits |
| 6 | Loss of data or delays in relaying data on the GTS |
| 7 | Routeing of data different from the routeing provided in the plan |
| 8 | Various malpractices |
| 9-14 | Reserved |
| 15 | Missing value |

0 35 031

Qualifier on monitoring results

| Code figure |  |
| --- | --- |
| 1 | Sufficient and all of acceptable quality |
| 2 | Sufficient but partly of acceptable quality |
| 3 | Insufficient but all of acceptable quality |
| 4 | Insufficient and of unacceptable quality |
| 5 | Some messages not complete |
| 6 | Suspect or wrongly coded groups could not be interpreted confidently |
| 7 | Gross coding errors |
| 8 | Transmission sequential order not observed |
| 9 | Report completely garbled and thus discarded |
| 10 | Deficiencies identified and rectified |
| 11 | Deficiencies identified but not rectified |
| 12 | Deficiencies not identified |
| 13 | Measuring errors |
| 14 | Mutual inconsistency |
| 15 | Temporal inconsistency |
| 16 | Forecast error |
| 17 | Bias |
| 18 | Improve system of quality control |
| 19 | Expand training programmes |
| 20-98 | Reserved |
| 99-122 | Not used |
| 123 | Missing value |

0 35 032

Cause of missing data

| Code figure |  |
| --- | --- |
| 1 | Data groups missing due to radio fading |
| 2 | Data groups missing due to outage of centre |
| 3 | Data groups missing due to outage of circuit |
| 4 | Non-implementation or maintenance of required RBSN density |
| 5 | Shortage of qualified staff to man stations |
| 6 | Lack of consumables |
| 7 | Instrument failure |
| 8 | Non-adherence to telecommunication procedures |
| 9 | Some observing programmes ceased |
| 10-14 | Not used |
| 15 | Missing value |

0 35 033

Observation and collection deficiency

| Code figure |  |
| --- | --- |
| 1 | No deficiency |
| 2 | Observations not made regularly |
| 3 | Observations not made at right time |
| 4 | Observations made but not disseminated |
| 5 | Observations made and sent to incorrect users |
| 6 | Collection not received |
| 7 | Collection transmitted late |
| 8 | Collection not transmitted |
| 9 | Difficulties in HF propagation and selection of suitable frequency |
| 10 | Difficulties in maintenance of communication equipment at remote stations |
| 11 | No alternative arrangement for routeing meteorological observation |
| 12-99 | Reserved |
| 100-122 | Not used |
| 123 | Missing value |

0 35 034

Statistical trends for availability of data (during the survey period(s))

| Code figure |  |
| --- | --- |
| 1 | Slight improvement |
| 2 | Significant improvement |
| 3 | Most significant improvement |
| 4 | Steady |
| 5 | Decreasing |
| 6 | Efforts required to improve night-time observations |
| 7 | Missing value |

0 35 035

# Reason for termination

| Code figure |  |
| --- | --- |
| 0 | Reserved |
| 1 | Balloon burst |
| 2 | Balloon forced down by icing |
| 3 | Leaking or floating balloon |
| 4 | Weak or fading signal |
| 5 | Battery failure |
| 6 | Ground equipment failure |
| 7 | Signal interference |
| 8 | Radiosonde failure |
| 9 | Excessive missing data frames |
| 10 | Reserved |
| 11 | Excessive missing temperature |
| 12 | Excessive missing pressure |
| 13 | User terminated |
| 14-29 | Reserved |
| 30 | Other |
| 31 | Missing value |

_________________

* The expression "at the station" refers to a land station or a ship.

** Tornado cloud or waterspout.

* Hail, small hail, snow pellets

* Hail, small hail, snow pellets.